2. TEXTUAL DOCUMENTATION

________________________________________________

2.1. Technical description – bridge structure

________________________________________________

2.1. Technical description

2.1.1. General project data

| EMPLOYER: | European Investment Bank |
| --- | --- |
| BENEFICIARY: | Željeznička infrastruktura Crne Gore AD Podgorica |
| STRUCTURE: | Bridge no. 101 at km 436+130.00 |
| PROJECT: | MAIN DESIGN FOR REHABILITATION OF BRIDGE no. 101 at km 436+130.00 |
| SECTION: | Vrbnica – Bar |
| CHAINAGE: | 436+130.00 |
| PROJECT STAGE: | Main design |
| NAME AND DESIGNATION OF PROJECT SECTION: | VOLUME 2.1 – STRUCTURAL DESIGN Bridge no. 101 at km 436+130.00 |
| PROJECT ORGANIZATION RESPONSIBLE FOR DRAFTING BRIDGE STRUCTURAL DESIGN: | Cestra d.o.o. Beograd Makenzijeva 57, 11118 Beograd |

2.1.2. 2.1.2.	Description of route location and alignment

The bridge is located in a valley. The bridge extends over a regulated bed of the Grabovac stream at an angle of 70 degrees. The terrain in the bridge area is stable.

The track on the bridge is in a transitional curve and is welded as CWR.

In the area of the structure there is space for construction site capacities.

2.1.3. Schedule 1 – Bridge layout plan from the ŽICG archives

2.1.4. Schedule 2 – Technical Report from the original design

Three versions of the original design from 1954 were found in the archives.

The first three versions of the original design do not correspond to the executed state of the structure.

The third version of the design is the closest to the executed state and it contains only the graphic documents while the Technical Report is missing. In the graphic documents file there is a drawing of the executed state on which the piles were crossed out and the new shape of the shallow founded piers with the altered foundation level 4m deeper than in the original version was drawn.

The third version of the Design served as the basis for drafting the Rehabilitation Design - executed state.

2.1.5. Schedule 3 – Assessment of bridge condition with proposed measures (Bridge

inspection report from 2015)

Taken from „The report on the state of the bridge: the bridge at km 436+130.00 the railway line Vrbnica – Bar” from 2015.

## General observation about the state of the bridge:

Measuring the dimensions of the structural elements and comparing with the dimensions given in layout sketches from "Report about the current state of bridges on the Vrbnica-Bar railway line" it was observed:

- dimensions of the bridge are in compliance with dimensions given in layout sketch from “Report about the current state of bridges on the Vrbnica-Bar railway line”,

- after removing the track ballast and measuring of all necessary dimensions,  thickness of the RC  slab was determined indirectly and it was found to be 25 cm (with a protective coating and waterproofing) in the middle of the slab.

Railway track on the bridge is in good condition. After removing the track ballast in the sleeper zone,thickness of the track ballast below the lower edge of sleeper was found to be 27 cm (in the axis of the railway track).

Damages that could reduce the capacity and stability of the bridge structure were not found during the detailed visual inspection of the bridge.

The observed damages and deficiencies of the structure which negatively affect the durability and functionality of the structure are the following:

- expansion joints are out of function (or does not exists), and that results in inflow of the water from the railway track to the backwalls, cap beams, abutmens and structure in the zone of the abutments,

- in the lower zone of RC slab there are zones without concrete protective layer with lot of concrete segregations zones,too.

## Proposed measures from the aspect of the bridge structure durability and functionality

- installation of the watertight expansion joints,

- cover all concrete surfaces with mortar and with protective coating in accordance to protect concrete and to extend it’s life.

2.1.6. Damage to the bridge and possible causes

- A detailed summary of damage with possible causes was given in the previous project stage:

- ˝Report on bridge condition: bridge at km 436+130.00 on Vrbnica – Bar railway line˝ from 2015 prepared by Pro-Inženjering d.o.o. in Belgrade

- A graphic representation of damage observed on bridge inspection is given in graphic documents of the design file in the drawing – Photograph of damage.

- Based on the "Report on bridge condition: bridge at km 436+130.00 on Vrbnica – Bar railway line˝ from 2015, the scope of damage was analysed alongside the methods for its rehabilitation. The volume of works and the investment value were based on the summary published in 2015 and further exploration works published in 2017.

2.1.7. Description of bridge structure – current condition

The bridge extends over a regulated bed of the Grabovik stream at an angle of 70 degrees.

The bridge RC structure is a simply-supported beam spanning 8.60m supported against concrete piers behind which there are parallel wing walls.

The bearing structure is a system of two beam girders measuring 80x90cm, which form T- sections with the RC bridge deck 20 cm thick. Main girders are placed at axial spacing of 220cm, while on the outside there are two cantilevers of the inspection walkways 120 cm long on both sides. On the bridge there are only terminal cross girders measuring 43x90cm in the cross-section, placed between main girders.

The main girders are supported against RC sleeper beams measuring 60x60cm over lead plates and anchors 2Ø36, forming fixed supports on abutments.

Abutments are massive and made of plain concrete. Foundation was completed on massive footings, which are buried at approximately 5.5m from the river bed under the bridge.

From the upper side of the bridge deck damp-proofing was placed not protected with a concrete cover. There is a drainage system on both sides of the bridge structure due to double-sided cross slope of the RC bridge deck.

The handrail on the footpath on the bridge structure was made of L steel sections with a welded joint. The handrail is 1100 mm high.

The following cables were laid on the bridge structure in the concrete duct on the left of the structure:

- main railway STA cable

- one optic cable with SM fibres in the PE pipe Ø 40 + a spare PE pipe  Ø 40

For the reinforced concrete elements the round concrete steel of Č37 (240/360Mpa) grade was used.

Concrete grades used in structural elements:

Main girders - MB200 - which corresponds to concrete grade C16/20 according to EN 1992-1-1

Cross girders - MB200 - which corresponds to concrete grade C16/20 according to EN 1992-1-1

Bridge deck - MB200 - which corresponds to concrete grade C16/20 according to EN 1992-1-1

Abutments - MB160 - which corresponds to concrete grade C12/15 according to EN 1992-1-1

Footings - MB140 - which corresponds to concrete grade C12/15 according to EN 1992-1-1

2.1.8. Static calculation - summary

2.1.9. Rehabilitation works

It is necessary to perform the following rehabilitation works on the bridge:

- removing the existing prefabricated cable duct and occasional local displacement of cables,

- controlled demolition of concrete in footpaths (while retaining the existing reinforcement) and dismantling of the existing handrail in the peripheral area in order to place a new cornice,

- controlled demolition of concrete in RC bridge deck and the parapet in order to install new expansion joints,

- replacing the present damp-proofing on the entire bridge,

- placement of new gullies,

- producing the base of plain concrete and fitting of new prefabricated cable ducts for cables on the left side of the bridge,

- installation of new waterproof expansion joints on both abutments,

- placement of cable duct lids made of corrugated sheet metal in places where expansion joints are put,

- repair works on concrete surfaces and anti-corrosive measures on reinforcement in places where concrete surfaces were damaged,

- producing new upper parts of cornices with the fitting of prefabricated cornices,

- placement of new handrails on footpaths,

- strengthening the connections of the pavement structure with abutments,

- applying protective concrete coating on all visible concrete surfaces after preparing the concrete surface using abrasive methods.

2.1.10. Technology of work execution

- REHABILITATION WORKS EXECUTED ON THE TRACK OR IMMEDIATELY BY THE TRACK

Given that each structure has different conditions for the transport of personnel, equipment, machinery and materials, the following principles were adopted:

- The construction site is erected in the area of the structure in case:

- there is an asphalt road or a good macadam road;

- there is free space for construction site capacities.

- The construction site is erected in the area of the nearest railway station and in that case the following applies:

- transport of personnel and material to the bridge is carried out using railway rolling stock during the whole time of works as per established procedures and possible time slots;

- make sure to provide a building for the security personnel, a building for workers' accommodation and the tool warehouse at the bridge location.

The following was adopted for bridge no. 101:

- there is no access road to the bridge;

- the central construction site should be placed in the Virpazar junction at km 434+000 as there is a road for truck and other vehicles leading there;

- the haulage of equipment, machinery and material from the Virpazar junction to the bridge is done with railway rolling stock;

- transport of personnel is possible via the asphalt road 300m from the bridge location.

- General

Here is a detailed description of the technology for the items of rehabilitation works executed on the track or immediately by the track. Other items of works are executed independently of railway traffic and are not in the high voltage area. The items of works for which the execution technology is described in detail are the following:

- Placement of new damp-proofing under ballast prism

Removal of the existing damp-proofing, sand blasting the concrete surface, applying repair mortar, producing new sprayed damp-proofing with a cover made of prefabricated sheets of extruded polystyrene.

- Placement of new gullies

Disassembly of the existing gullies, treating the area with repair mortar, treating the base made of plain concrete under prefabricated drain flumes in the area of gullies, applying damp-proofing and fitting of new gullies.

- Placement of new bridge expansions

Treating concrete services, fitting of new expansion joints and placing lids of corrugated sheet metal on cornices in places where expansion joints are put.

- Repairing cantilever cornices on footpaths

Machine removal of the top and lateral surface of concrete cornices, sandblasting the concrete and the rebar, producing new upper sections of the cornices and fitting prefabricated cornices.

- Placement of new handrail on footpaths

Removal of existing handrail on footpaths, the placement of new handrail on previously repaired cornices.

- New prefabricated ducts for cables

- removing the existing prefabricated cable ducts and their lids along the entire bridge length.

- producing the layer of plain concrete as the base under prefabricated ducts. The concrete base is built along the entire bridge length, on the left side of the bridge.

- placing new cable ducts and their lids along the entire bridge length.

It is essential to make new damp-proofing and new expansions in a high-quality manner so as to eliminate harmful effects of water on the structure long term. Placement of damp-proofing and the repair mortar coating must be done adhering to temperature requirements, humidity and hardening times. It is also important to avoid frequent resumption of works when damp-proofing as much as possible.

The above-mentioned items of works are executed under special railway traffic management schemes as follows:

- item 1 is executed under traffic closure and voltage shutdown in the maximum period of time;

- items 2 and 3 are executed under “railway closure for construction works” scheme in the maximum period of time in night-time or day-time slots with or without power shutdown;

- items 4, 5 and 6 are executed under low-speed traffic scheme (30 km/h) without power shutdown.

Works defined under items 2 through 6 may be executed under railway traffic scheme a) or b) if this does not interfere with the works defined under item 1 in the Description of works.

- Track works (the bridge and the bridge area)

For the execution of works under items 1, 2 and 3, it is necessary to remove the rails, sleepers and the crushed stone ballast all the way to the damp-proofing layer.

Here is the description of items of track works that need to be executed.

- Works to be executed before traffic closure:

- assembly and disassembly of rail anchors in the track area after abutments;

- assembly and disassembly of sleeper anchors in the track area after abutments;

- cutting rails in the embankment at 10m from abutments and at every 22.5m on the bridge with rail drilling and fitting of rail connectors.

- Works executed under traffic closure and power shutdown

- track disassembly (rails, check rails, sleepers, ballast prism) of one 15m span;

- assembling the track using existing rails, the new crushed stone ballast, new wooden sleepers and new rail fasteners.

- Works executed after construction works in the track area

- welding rails as CWR, placement of check rails;

- regulating the track by direction and the level line according to current condition elements.

- Strengthening the connections of the pavement structure with abutments

- Chasing is performed on the cross girder and the sleeper beam. Chasing is performed while retaining the existing reinforcement of the cross girder and the sleeper beam;

- Cleaning concrete surfaces and applying SN binder (old-new) and anti-corrosive protection on the existing reinforcement;

- Fitting new anchors;

- Concreting and returning into original condition - concrete grade (C35/45). In concreting the part of the sleeper beam and the part of the cross girder, apply monolithic concreting.

- Conclusion

- The prerequisite for quality execution of damp-proofing works, gullies and expansion joints is to adhere to installation requirements and work carefully without any unreasonable demands to shorten the time required for such works.

- With good preparation and good management of work train traffic, sufficient number of pieces of construction machinery and experienced and qualified workforce, it is possible to repair damp-proofing on the deck on 15m of the bridge under one-day continuous closure (24h) and go to low-speed traffic afterwards.

This applies to the Vrbnica-Podgorica railway section for the period May-September and the Podgorica-Bar section for the period April-October.

- The Contractor shall, as per design recommendations and his technical and technological capacities determine which periods of railway closures are necessary and submit a timely request to ŽICG for railway closures longer than those approved on a daily basis.

- To the extent possible, traffic closures should be employed simultaneously on two or more bridges which are at the same inter-station distance.

- In bridges with spans over 15m, repeated use of long closures is necessary, although it is acceptable to employ such closures with breaks in between closures that are not longer than 2 days.

### 2.1.11.  The list of templates used for the creation of technical documentation

- The design engineer used the following documents for the creation of the rehabilitation project:

- The original design - Main design of RC bridge L=8.00m over stream "Grabovik" at km 31+945 of railway line  Titograd - Bar (developed by Institute for designing Belgrade, 1954.); - three versions of the project;

- Bridge condition report: Bridge in km. 438+130.00 of railway line Vrbnica - Bar (developed by Pro-inženjering, 2015.);

- Geotechnical elaborate (2018.);

- Geodetic survey (2018.).

### 2.1.12.  List of applied regulations

LAWS AND RULEBOOKS REGARDING THE CONTENTS OF THE TECHNICAL DOCUMENTATION

- Railway law. Official Gazette of RMN, N° 27/2013;

- Law on safety, organization and efficiency of rail transport of Montenegro of 27/12/2013, in force since January 2014;

- Law on spatial development and construction of structures. Official Gazette of RMN, N°51/08, 40/10, 34/11, 47/11, 35713, 39/13;

- Law on construction products N° 18/2014;

- Law on geological researches. Official Gazette of RMN, N° 28/93, 27/94, 42/94, 26/07;

- Law on occupational safety. Official Gazette of RMN, N° 34/2014;

- The Rulebook on content and production of technical documentation - Official Gazette of RMN, N° 23/14, 32/15, 75/15;

RULEBOOKS FOR THE DESIGN

- (316) Rulebook on technical standards for determining the size of the load and categorization of railway bridges, culverts and other structures on railway lines. Edition 1992

EN STANDARDS

- MEST EN 1990:2013-Eurocode - Basis of structural design.

- MEST EN 1990:2013/NA:2013- Eurocode - Basis of structural design - National Annex.

- MEST EN 1991-1-1:2017/NA:2017- Eurocode 1: Actions on structures - Part 1-1: General actions - Densities, self-weight, imposed loads for buildings - National Annex

- MEST EN 1991-1-3:2017-Eurocode 1 - Actions on structures - Part 1-3: General actions - Snow loads.

- MEST EN 1991-1-3:2017/NA:2017- Eurocode 1 - Actions on structures - Part 1-3: General actions - Snow loads - National Annex

- MEST EN 1991-1-4:2016-Eurocode 1: Actions on structures - Part 1-4: General actions - Wind actions.

- MEST EN 1991-1-4:2016/NA:2016- Eurocode 1: Actions on structures - Part 1-4: General actions - Wind actions - National Annex.

- MEST EN 1991-1-5:2017/NA:2017- Eurocode 1: Actions on structures - Part 1-5: General actions - Thermal actions - National Annex

- MEST EN 1992-1-1:2017/NA:2017 -  Eurocode 2: Design of concrete structures - Part 1-1: General rules and rules for buildings - National Annex

- MEST EN 1997-1:2017- Eurocode 7: Geotechnical design - Part 1: General rules - National Annex;

- MEST EN 1997-1:2017- Geotechnical design - Part 1: General rules;

- MEST EN 1998-1:2015 - Eurocode 8: Design of structures for earthquake resistance - Part 1: General rules, seismic actions and rules for buildings.

- MEST EN 1998-1:2015/NA:2015 - Eurocode 8: Design of structures for earthquake resistance - Part 1: General rules, seismic actions and rules for buildings - National Annex

- SRPS EN 1991-1-5:2012 - Eurocode 1: Actions on structures - Part 1-5

- SRPS EN 1991-1-1:2012 - Eurocode 1: Actions on structures -Part 1-1

- SRPS EN 1991-1-3:2012 - Eurocode 1: Actions on structures -Part 1-3

- SRPS EN 1992-1-1:2015 - Eurocode 2: Design of concrete structures: Part 1-1

- SRPS EN 1997-1:2004 - Eurocode 7: Geotechnical design - Part 1

- HRN EN 1991-1-7:2012 - Eurocode 1: Actions on structures - Part 1-7

- HRN EN 1991-1-7:2012/Cor.1:2015 - Eurocode 1: Action on structures -Part 1-7

- HRN EN 1991-1-7:2012/A1:2015 - Eurocode 1: Action on structures -Part 1-7

- HRN EN 1991-2:2012 - Eurocode 1: Action on structures -Part 2

- HRN EN 1992-2:2013 - Eurocode 2: Design of concrete structures -Part 2

- HRN EN 1998-2:2011 - Eurocode 8: Design of structures for earthquake resistance - Part 2

- HRN EN 1998-3:2011 - Eurokod 8: Design of structures for earthquake resistance -Part 3

- HRN EN 1998-3:2011/Cor.1:2014 - Eurokod 8: Design of structures for earthquake resistance -Part 3

- HRN EN 1998-5:2011 - Eurokod 8: Design of structures for earthquake resistace - Part 5

2.2. Technical description – superstructure

________________________________________________

2.2	 TECHNICAL DESCRIPTION - SUPERSTRUCTURE

2.2.1	GENERAL

Within the Main Design for the rehabilitation of bridge no. 101 works on the track superstructure are planned, which are to serve as bridge rehabilitation. Technical documents for bridge no. 101 analysed a track section 28.60m long, which includes the entire bridge length and 10m before and after the bridge.

Based on the Conclusion of the Inspection Report on the condition of bridge 101 superstructure and establishing the current condition in the drafting period of the relevant technical documents, works to be done on the bridge track superstructure were defined. The track on the bridge structure is in a curve (R=800 L=80) placed on a part of a transition curve and is welded as CWR.

The Measures regarding the bridge superstructure with the drafting of the Priced BoQ, Technical Specifications, Superstructure Static Calculation and the Plan for track development were defined.

Total value of superstructure works for bridge no. 101 is €23,341.42.

2.2.2	MEASURES REGARDING THE SUPERSTRUCTURE

For the execution of rehabilitation works (placing new damp-proofing under the ballast prism, installation of new gullies and expansion joints) the following works on the superstructure were planned, where the bridge length and the length on the embankment of 10 m from abutments, before and after the bridge are taken into account:

- arresting CWR before and after the bridge with the installation of rail anchors;

- disassembly of the existing track in segments up to 22.5m accompanied by the removal of the track, whereby it is necessary to cut the existing track and drill rails for the connections at the joint, any disassembly of fasteners and sleepers as per track technology used for the removal of the track;

- removal of the current crushed stone ballast all the way to the damp-proofing protective layer;

- installation of the track in segments up to 22.5m with new crushed stone, new sleepers and new fasteners, with the fitting of tie plates and the necessary fasteners for joining running rails;

- regulating the track by direction and the level line according to current condition elements;

- disassembly of rail joints on running rails before welding;

- welding the track as CWR;

- disassembly of rail anchors;

- final track regulation.

Bridge superstructure works for bridge rehabilitation, particularly the disassembly and the assembly of the existing track, are executed under traffic closure while low-speed traffic scheme should be employed on the bridge section during rehabilitation works.

2.2.3	STATIC CALCULATION-SUMMARY

The basic elements of the superstructure used in the calculation of track stresses and stability are as follows:

- 49 E1 rails, grade R260 (900A)

- wooden sleepers L=260cm, at axial spacing of 60cm

- "K"-type fasteners.

The bridge structure is made of reinforced concrete, it has a slab in the cross-section, comprises a simply-supported beam spanning L=8.60m supported against concrete piers behind which there are parallel wing walls. The bearing structure is a system of two main beam girders, an RC bridge deck and cantilevers of the footpaths on both sides. The bridge extends over a regulated bed of the Grabovac stream at an angle of 70 degrees. The track on the bridge structure is in a curve (R=800 L=80) placed on a part of a transition curve and is welded as CWR.

Having in mind the provisions of the Rulebook on railway superstructure maintenance (“Official Journal of Montenegro, no. 42/2016 of 11 July 2016), the applicable EN standards and UIC publications, and the view that there is no relative displacement between the bridge and the continuous welded track (Study of various methods adopted by world railways to continue LWR over bridges), the calculation was made accounting for the effect of temperature changes in the rail without the effect of bridges on the rail (as on an open railway, i.e. the track was observed on the embankment).

Characteristic results

a) For a continuous welded track made of 49E1 the maximum compression strength is:

N = - 661.50kN

b) For a continuous welded track made of 49E1 the maximum tensile strength is:

N = 817.14kN

c) The inspection of track stability against buckling in the curve with a radius R=800m on the bridge in extreme summer temperatures employing the Mischenko energy model was made.

They obtained critical lateral resistance values of q = 79.25 N/cm < 90 N/cm for which it is not necessary to fit sleeper anchors, that is a well-tamped ballast shall provide the stability of track against buckling.

d) Inspection of crack size on rail cracking in winter determined that on rail cracking, the crack size shall be 2.52cm<10cm, i.e. traffic safety shall not be at risk in case of rail cracking.

e) In order to reduce the impact of CWR on bridge piers, longitudinal resistances were reduced on the total bridge length of 8.60m. On the ground this reduction in resistances is achieved by fitting trimmed tie plates (by trimming the shorter point by 4 mm) in the distance of 8.60m (15 sleepers).

2.2.4	SOLUTIONS FOR TRACK DEVELOPMENT

Based on the performed analysis and the obtained calculation results, the track should be developed in the following way meeting the following requirements:

- The substructure before and after the bridge must be completely and properly executed (cross-section, drainage, stabilization).

- Crushed stone ballast before, after and on the bridge must be uncontaminated, having the recommended quality and grain size distribution, with a ballast prism intended for CWR. On sections where rail anchors are placed, additional tamping of ballast prism should be performed.

- Before welding, the track (on the bridge and outside the bridge) must be completely regulated by direction and the level line as per the elements of the current condition.

- The calculation was made under the condition that both the railway line and the track on the bridge are welded as CWR.

- The required temperature for CWR tempering is +230C ± 30C.

- Before CWR arrest, 74 pieces of rail anchors are fitted on about 50m of the track before and after the bridge, which shall be dismantled after final welding.

- In order to reduce the impact of CWR on bridge piers, longitudinal resistances were reduced on the total bridge length of 8.60m. On the ground this reduction in resistances is achieved by fitting trimmed tie plates (by trimming the shorter point by 4 mm) in the distance of 8.60m (15 sleepers).

- It is not necessary to place sleeper anchors on the bridge, i.e. the track in the curve with R=800m having a well-tamped ballast provides stability of track against buckling.

- Permanent labels for monitoring CWR longitudinal and transversal displacement (in the bridge area) should be put in the following places:

- at the first sleeper after the bridge

- at the first sleeper before the bridge.

Labels are buried on both sides of the track on stable soil.

Labels are placed before completing CWR formation, and are calibrated immediately after CWR tempering in the presence of the Supervisor.

Chainage labels can be used as permanent labels for CWR monitoring, provided they are in the immediate vicinity (within 3m) of the places where labels are placed. Permanent labels are not included in the Priced Bill of Quantities.

- All the necessary track maintenance works should be done on time, in a quality manner and within the allowed temperature ranges. It is also necessary to ensure enhanced supervision on the section at extreme rail temperatures (below  -10°C and over +35°C).

3. NUMERICAL DOCUMENTS

________________________________________________

3.1. Static calculation for bridge structure

________________________________________________

STATIC CALCULATION CONTENTS

    - INPUTS

      - Structure layout - main design (1954)

      - Data from the main design (1954)

      - Cross-section of the structure - newly designed (2017)

      - Material characteristics

        - Concrete characteristics

        - Reinforcement characteristics

        - Foundation soil characteristics

      - Partial safety factors related to materials

      - Calculation models

        - First calculation model

        - Second calculation model

      - List of loads

        - Basic calculation (ULS and SLS)

        - Seismic calculation (ULS)

        - Forming combinations of loads

    - LOAD ANALYSIS

      - Permanent loads Gki,S

        - Dead load Gk,1

        - Permanent load on the bridge Gk,2

        - Earth pressure Gk,3

        - The effects of concrete creep and shrinkage S

      - Variable actions Q,T,F,s

        - Movable load from the train Qk,1

        - Requirements for static or dynamic calculation

        - Dynamic coefficients

        - "Empty wagons“ loading model qk,2

        - Imposed load on inspection walkways qfk

        - Lateral force Qsk

        - Forces arising from train breaking and train starting Qlak,qlak,Qlbk,qlbk

        - Temperature effects Tk

        - Wind loads Fwk

        - Snow loads sk

        - Effect of horizontal forces in CWR qcwr

      - Seismic forces AE

        - Seismic forces – bridge structure AEk

        - Seismic earth pressure AEd

      - Accidental actions Ad

        - Effects of  derailment on the bridgeQA1d,qA1d,qA2d

    - POSITIONS OF LOADS

    - PRINCIPLES OF LOAD COMBINATIONS

      - Summary of loads

      - Excerpt from EN1991-2 and EN1990

      - Ultimate limit state ULS

        - Combination of actions for permanent and transient calculation situations (basic combinations)

        - Combinations of actions for accidental calculation situations

        - Combinations of actions for seismic calculation situations

        - Combinations of actions for fatigue calculation (approximate method)

      - Serviceability limit state SLS

        - Combination of actions for irreversible serviceability limit state

        - Quasi-constant combination of actions for long-term actions and structure appearance

    - SERVICEABILITY LIMIT STATE (SLS)

      - Pavement structure sagging

      - Vertical deformations of the bridge pavement structure

        - Rotations in the ends of the bridge pavement structure

        - Vertical motion in the ends of the bridge pavement structure

      - Torsion of the bridge pavement structure

      - Longitudinal motion in the ends of the bridge pavement structure

        - Longitudinal motion in the ends of the bridge pavement structure due to LM71

        - Longitudinal motion in the ends of the bridge pavement structure due to breaking and starting forces

      - Transverse deformations of the bridge pavement structure

      - Lateral vibrations in the bridge pavement structure

      - Calculation of cracks in main girders

      - Calculation for new expansion joints

        - Approving expansion joints

    - ULTIMATE LIMIT STATE (ULS)

      - Main girders - beam girders with "T" cross-section

        - Static effects in main girders

        - Diagrams of necessary reinforcement in main girders

        - Existing reinforcement in main girders

        - Checking shear capacity in main girders

        - Fatigue calculation

        - Summary of obtained results and conclusion on main girders bearing capacity

      - RC bridge deck

        - Static effects in the bridge deck

        - Diagrams of necessary reinforcement in the bridge deck - movable LM71

        - Existing reinforcement in the bridge deck

        - Diagrams of necessary reinforcement in the bridge deck - movable D4

        - Summary of obtained results and conclusion on the bridge deck bearing capacity

      - Cantilever of  (inspection) walkway

        - Cantilever load

        - Static effects in the cantilever

        - Combination of loads for cantilever design

        - Diagrams of necessary reinforcement in cantilever

        - Existing reinforcement in cantilever

        - Summary of obtained results and conclusion on cantilever bearing capacity

      - Abutment S1 - design abutment with fixed support

        - Checking normal stress in cross-section 1-1

        - Checking shear stress in cross-section 1-1

        - Checking normal stress in cross-section 2-2

        - Checking shear stress in cross-section 2-2

        - Checking normal stress in cross-section 3-3

        - Checking shear stress in cross-section 3-3

        - Checking stresses in footing joint

        - Summary of obtained results and conclusion on abutment S1 bearing capacity

      - Fixed support on abutment S1 - design fixed support

        - Calculation for anchors joining the sleeper beam and main girders

        - Summary of obtained results and conclusion on fixed support bearing capacity

      - New handrail on (inspection) walkway

        - Calculation for handrail post

        - Calculation for handrail bar

        - Calculation for the connection of handrail post to frontal slab

        - Calculation for the anchors connected to frontal slab

    - SEISMIC CALCULATION ACCORDING TO EN1998 (ULS)

      - Longitudinal and transversal direction - X and Y direction

        - Calculation model for X and Y directions

        - Oscillation periods, participation of mass, seismic forces

      - Abutment S1 - design pier with fixed support

        - Checking normal stress in cross-section 1-1

        - Checking shear stress in cross-section 1-1

        - Checking normal stress in cross-section 2-2

        - Checking shear stress in cross-section 2-2

        - Checking normal stress in cross-section 3-3

        - Checking shear stress in cross-section 3-3

        - Checking stresses in footing joint

        - Summary of obtained results and conclusion on abutment S1 seismic bearing capacity

      - Fixed support on abutment S1 - design fixed support

        - Calculation for anchors joining the sleeper beam and the bridge deck due to seismic actions

        - Summary of obtained results and conclusion on fixed support bearing capacity

    - CALCULATION FOR STRUCTURE STRENGHTENING

      - Strengthening fixed supports on all piers

        - Calculation for strengthening the connection between the sleeper beam and main girders