2. TEXTUAL DOCUMENTATION

________________________________________________

### Technical description – bridge structure

| EMPLOYER: | European Investment Bank |
| --- | --- |
| BENEFICIARY: | Željeznička infrastruktura Crne Gore AD Podgorica |
| STRUCTURE: | BRIDGE no. 70, at km 366+293.45 |
| PROJECT: | MAIN DESIGN FOR REHABILITATION OF BRIDGE at km.366+293.45 |
| SECTION: | Vrbnica - Bar |
| CHAINAGE: | 366+293.45 |
| PROJECT STAGE: | Main design |
| NAME AND DESIGNATION OF PROJECT SECTION: | VOLUME II – STRUCTURAL DESIGN Bridge no 70, AT km 366+293.45 |
| PROJECT ORGANIZATION RESPONSIBLE FOR DRAFTING BRIDGE STRUCTURAL DESIGN: | Cestra d.o.o. Beograd Makenzijeva 57, Beograd 11000 |

### Description of route location and alignment

The bridge is located at chainage km .366+293.45 on the Vrbnica-Bar railway line. The bridge structure is a continuous beam structure on five spans of 2×12.0+18.0+2×12.0m, i.e. in total length of L=66.0m. The bridge extends over a ravine app. 19m deep, the terrain is a natural steep slope.

The tracks on the bridge are designed in two transition curves and is straight for only 34cm, i.e. between the begining of a transition curve and the end of a transition curve at the pier S2. The bridge structure is straight from the abutment S1 to pier S2 and the tracks deviate 5.3cm at their maximum, whereas the bridge axis and the track axis coincide at the  axis of the pier S2. The structure of the bridge breaks chordally from the pier S2 axis towards abutment S5, so that the track axis and the bridge axis coincide. The largest deviation of the track axis and the bridge axis in that area is 4.0cm. The level line is lowered 23‰towards Podgorica. The tracks are welded as CWR.

### Schedule 1 – Bridge layout plan from the ŽICG archives

### Schedule 2 – Technical Report from the original design

The main structural design was done by the Association of Yugoslav Railways, Designing Institute, Belgrade, 1971.

The main design dates back to 1971.

### Schedule 3 – Assessment of bridge condition with proposed measures (Bridge inspection report 2015)

Taken from „The report on the state of the bridge: the bridge at km 366+293.45 the railway line Vrbnica – Bar” from 2015.

## General observation about the state of the bridge

By measuring the dimensions of structural elements and by comparison with layout sketches from the “Report about the current state of bridges on the Vrbnica-Bar railway line”, the following is concluded:

- the general bridge dimensions comply with the dimensions from the "Report about the current state of bridges on the Vrbnica-Bar railway line",while the certain differences in dimensions as well as missing dimensions are shown in appendix of this Report,

- after removing the ballast and measuring of all necessary dimensions,  thickness of the RC slab was determined indirectly and it was found to be 24 cm (in the middle of the slab with a protective coating and waterproofing and with presumed both sided incline of the slab of 2%). Insight in to the part of design documentation showed that there are differences between measured values of slab thickness and values in the design documentation. It is necessary to do a more detail insight in to the design documentation and to determine the thickness of the slab in the static analysis of the RC slab.

Railway track on the bridge is in a slight curve and carried out in continuous welded rail (CWR). Sleepers are with different age,some with cracks and in the bad condition,rail fastening is loose on some places and with missing elastic washers beneath the spikes. There is the only one safety rail on the bridge,on the left side next to the left rail,with loose rail fastening on all connections to the sleepers,as well as with missing bolts in the places of rail splices (continuing). Ballast on the bridge is unclean. It’s necessary to replace all worn-out and cracked sleepers and to fasten all rail fastenings. One more safety rail (next to right main rail) must be installed and proper connection to the sleepers of existing one (left safety rail) should be executed. After removing the ballast in the sleeper zone, minimum thickness of the track ballast below the lower edge of sleeper was found to be 33 cm (in the axis of the railway track).

Damages that could reduce the capacity and stability of the bridge structure were not found during the detailed visual inspection of the bridge.

The observed damages and deficiencies of the structure which negatively affect the durability and functionality of the structure are the following:

- segregations of the concrete of the both main girders in the span S3-S4 as well as in the zone of abutment S1 (just the right main girder) with exposed and corroded reinforcement,

- increased segregation of the concrete of the middle cross girder in the span S3-S4 with exposed and corroded reinforcement,

- damages of concrete in the lower zone of the sidewalk cantilevers around the drainage pipes of the installation channels – increased degradations of concrete with exposed and heavily corroded reinforcement as well as with increased water leakings. Also,larger degradation of concrete in the lower zone of the left sidewalk cantilever in the zone of the abutment S1 have been observed,with exposed and heavily corroded reinforcement,

- bad condition of concrete in the lower zone of overhead contact line (OCL) cantilevers. Due to a missing drips on the edges of cantilevers,water is soaking the concrete in the lower zone and in the places of anchor bolts there are bigger degradations of concrete with exposed and corroded reinforcement. Steel angle sections on connections between anchor bolts and cantilever are almost without contact concrete surface which jeopardize the connections and stability of the overhead contact line (OCL) columns,

- lack of waterproof expansion joints on sidewalk cantilevers at the bridge ends while the rest are damaged which results in increased water drain from the track to cantilevers, main girders and abutments,

- larger zone of concrete segregation of the right wing wall sidewalk cantilever of the abutment S1 with exposed and corroded reinforcement,

- damages of concrete on the left side of the pier S2 due to rock impacts,with exposed and corroded reinfrocement,

- larger degradations of concrete of the piers S3 and S5,in the zone of connection between pier and foundation,with exposed and corroded reinforcement,

- segregation of concrete on lot of places of the pier S4 – on sides,faces as well as on the bearing beam,

- right wingwall of abutment S6 is too short with a gap between lower edge of wingwall and terrain,

- prefabricated RC covers of the channels for installations are damaged on the lot of places with exposed and corroded reinforcement.

## Proposed measures from the aspect of the bridge structure durability and functionality

- rehabilitation of concrete of the main girders and cross girders in all places mentioned above,

- rehabilitation of concrete of the sidewalk cantilevers in the zones of drainage pipes of installation channels as well as rehabilitation of concrete in the lower zone of the left sidewalk cantilever in the zone of the abutment S1. Repairing of the waterproofing in this areas,

- rehabilitation of concrete in the lower zone of the overhead contact line (OCL) cantilevers with making of drips on the edges,

- incorporation (installation) of new waterproof expansion joints in the whole length in dilatation zones of abutments S1 and S6,

- rehabilitation of concrete of the piers S3 and S5 in the zone of connection between pier and foundation as well as rehabilitation of damaged concrete of the pier S2 due to rock impact,

- extend the existing right wingwall of abutment S6,

- replace of the damaged prefabricated RC covers of the channels for installations.

For these works it is necessary to make the technical documentation which must include a technical description of works, the technology of the works, estimated bill of quantity, and the necessary technical details.

Urgent measures on the bridge construction

Bad condition of concrete in the lower zone of overhead contact line (OCL) cantilevers. Due to a missing drips on the edges of cantilevers, water is soaking the concrete in the lower zone and in the places of anchor bolts there are bigger degradations of concrete with exposed and corroded reinforcement. Steel angle sections on connections between anchor bolts and cantilever are almost without contact concrete surface which jeopardize the connections and stability of the overhead contact line (OCL) columns.

It’s urgent to do a rehabilitation of concrete of OCL cantilevers in the lower zone,around the anchor bolts for connection and below steel angle sections,too.

### Damage to the bridge and possible causes

A detailed summary of damage with possible causes was given in the previous project stage:

˝Report on bridge condition: bridge at km 366+293.45 on Vrbnica – Bar railway line˝ from 2015, which was prepared by Pro-Inženjering d.o.o. in Belgrade.

A graphic representation of damage observed on bridge inspection is given in graphic documents of the design file in the drawing – Photograph of damage.

Based on the “Report of bridge condition: bridge at km. 366+293.45 on Vrbnica – Bar railway line˝ from 2015 the scope of damage was analysed alongside the methods for its rehabilitation. Scope of works and investment value were based on an inspection performed in 2015.

### Description of bridge structure – current condition

The bridge structure is a continuous beam structure with five spans of total length of L=2×12.0+18.0+2×12.0=66.0m in the vicinity of Kolašin. The bridge extends over a dry valley. The tracks on the bridge go from one transition curve into another transition curve and are in a straight line in just one small part near the pier S2. The bridge structure is partially in a straight line and partially breaks chrodally and follows the tracks. The track are produced as a CWR. The total width of the bridge is 6.30m, with 75 cm wide footpaths on each side of the bridge.

The bridge structure comprises two reinforced concrete beams with a T-section. Longitudinal girders are axially distanced by 2.75m from one another. The total height of girders is 2.00m, and the slab is 25cm thick in the middle. The bridge deck is haunched on the underside and designed in a dual slope of 2%. Cantilevers of footpaths are 1.50m and are haunched with maximum thickness of 30cm. Longitudinal girder webs are constant - their width being 55 cm.

Girders are connected with cross members designed on a grid of 6.0m. Intermediate cross girders are 30cm thick, whereas the end supports are 50cm, and intermediate supports are 60cm and 70cm thick. In terms of static, the structure is a frame structure made of continuous beams. Piers are fixed to the superstructure and wedged into footings, except for the pier S2 which is connected to the footing though a joint. The abutment S1 has a moving bearing in the form of a pendulum, and the abutment S6 has a fixed support.

Girders, bridge deck and cross girders were made using the MB 300 concrete grade. Pendulum on pier S1 and the joint on the pier S2 were made of MB400. Piers and foundation beds were made of MB 300, whereas abutments and foundations were made of MB160. Bearing beams were made of MB 220 concrete.

Rebar was made of Č-37 steel grade.

Piers have a square cross-section, S2 and S5 measuring b×d=3.20×0.6m, S3 and S4 measuring b×d=3.20×0.7m. Abutments basis measure b×d=1.55×4.0m – pier S1 and b×d=2.50×4.50m – pier S6. Foundation was planned to be don on a limestone mass.

Structure layout

Longitudinal section

Structure base

##### Current condition:

The track on the bridge is located in a curve and was produced as a CWR. The track axis deviates from the bridge axis which is broken in the horizontal plane above the pier S2. The track is a closed one with a crushed stone ballast on the bridge deck. Damp-proofing was placed on the upper deck with a concrete cover of 5cm.

At the ends of the bridge deck, there are footpaths 75cm wide. There are cable ducts on footpaths covered with prefabricated RC covers that form the footpath floor. Drainage on the structure is handled using a slope of the bridge deck and gullies installed on both sides as part of footpaths. The handrail on the bridge structure was made with welded pipe sections. The handrail is 1100mm high.

RC cantilevers are planned for the overhead line poles.

existing cross-section

- Newly designed condition:

The rehabilitation design provides for damp-proofing replacement on the bridge deck, reconstruction of the drainage system and reconstruction of footpaths.

Due to damp-proofing replacement, removal of crushed stone and the the existing damp-proofing cover, traffic closure is planned in the course of such works. Before damp-proofing placement, and if necessary, the bridge deck should be fixed using repair mortar. The new damp-proofing is a spray-type waterproofing.

The existing gullies shall be replaced by new ones. Gullies will be extended up to the bottom edge of the girder and are supported against girder webs in all aspects as per details given in Graphic Documents and the scale with Drainage Design.

Footpaths shall be reconstructed by adding prefabricated cornices, replacing the existing prefabricated slabs with new ones and repairing inner surfaces of the cable ducts. Damp-proofing and duct filling in the footpath is also planned. The filling for the footpath shall be made of sand placed in two layers with cable ducts. New footpath details are given in Graphic documents. The exiting drainage outlets in cable ducts on footpaths shall be closed and new drain tubes installed as per new layout in all things according to the details given in Graphic documents.

The newly designed condition plans for the strenghtening of the abutment S6 and for the widening of the footing due to its poor bearing capacity.

In addition, the footpath handrail and expansions joints should also be replaced.

newly designed cross-section

- Superficial damage repair

Superficial damage repair on outer girder surfaces in the form of crack repair by injection and repair of damaged covers. Following superficial damage repair works, it is necessary to protect all the outer girder surfaces with protective coatings.

The planned intervention areas for superficial damage repair are shown in Graphic documents.

- Interventions on the bridge during exploitation

According to the existing documents obtained from ŽICG, we established that no changes occurred in the course of bridge exploitation.

### Static calculation - summary

### Technology of work execution and rehabilitation works

The construction site provides:

- storage space for the tools and equipment;

- storage space for the material;

- enclosed and secured space for combustible materials and fuels;

- office space for the Contractor and the Engineer;

- compounds for the workers;

- area for the machines;

- other.

Given that each structure has different conditions for the transport of personnel, equipment, machinery and materials, the following principles were adopted:

- The construction site is formed in the area of the structure in case:

- there is an asphalt road or a good macadam road;

- there is free space for construction site capacities.

- The construction site is formed in the area of the nearest railway station and in that case the following applies:

- transport of personnel and material to the bridge is carried out using railway rolling stock during the whole time of works as per established procedures and possible time slots;

- providing a building for the security personnel, a building for workers' accommodation and the tool warehouse at the bridge location.

The following was adopted for bridge M70:

- there is no access road to the bridge

- central construction site should be formed at Trebešica junction at km 358+800

- transport of personnel, equipment, machinery and material from Mojkovac station to the bridge is carried out using railway rolling stock

- personnel can be transported to the bridge by train from Trebešica junction is carried out using railway rolling stock..

- Superstructure

The items of works for which technology of execution is described in detail are as follows:

  - Placement of new damp-proofing under the track prism

Removal of a cover (fine-grained concrete) and removal of existing damp-proofing, sandblasting of concrete surfacing, applying repair mortar, producing new spray damp-proofing with a cover made of prefabricated sheets of extruded polystyrene.

  - Placement of new gullies

Disassembly of existing gullies, treatment of the area with repair mortar, applying damp-proofing and installation of new gullies.

  - Placement of new bridge expansion joints

Disassembly of existing expansions, treatment of concrete surfaces, fitting new expansion joints.

  - Repairing cable ducts

Removing sand from the ducts, local dislocation of SS and TT cables, sandblasting of concrete surfaces, applying repair mortar, installation of drain tubes and applying spray damp-proofing.

  - Repair of cornices on footpaths

Machine removal of the top and lateral surface of concrete cornices, sandblasting the concrete and rebar, producing new section of a cornice with a drip cap.

  - Placement of new handrail on footpaths

Removal of existing handrail on footpaths, placement of new handrail onto previously repaired cornices.

It is essential execute new damp-proofing, new expansion joints and new gullies in a high-quality manner so as to eliminate harmful effect of water on the structure long term. Placement of damp-proofing and the repair mortar coating must be done adhering to the temperature, humidity and hardening times. It is also important to avoid frequent resumption of works when damp-proofing as much as possible.

The above-mentioned items of works are executed under special railway traffic management schemes as follows:

- items 1 and 2 in bridges with gullies at the structure axis are executed under traffic closure and voltage shut-down for a maximum period of time;

- items 2 and 3 are executed under “railway closure for construction works” scheme in the day-time slots (e.g. 1130 to 1630 ) with or without power shutdown;

- items 4, 5 and 6 are executed under low-speed traffic scheme (30 km/h) without power shutdown.

Works defined under items 2 through 6 may be executed under railway traffic scheme a) or b) if this does not interfere with the works defined under item 1 in the Description of works.

- Track works (the bridge and the bridge area)

For the execution of works under items 1, 2 and 3, it is necessary to remove the rails, sleepers and crushed stone ballast all the way to the damp-proofing layer.

Here is the description of items of track works that need to be executed.

- Works to be executed before traffic closure:

- assembly and disassembly of rail anchors in the track area after abutments;

- assembly and disassembly of sleeper anchors in the track area after abutments;

- cutting rails in the embankment at 10m from abutments and at every 22.5m on the bridge with rail drilling and fitting of rail connectors.

2)	Works executed under traffic closure and power shutdown:

- track disassembly (rails, check rails, sleepers, ballast prism) in a field of 15m;

- assembling the track using existing rails, the new crushed stone ballast, new wooden sleepers and new rail fasteners.

3)	Works executed after construction works in the track area:

- welding rails as CWR, placement of check rails;

- regulating the track by direction and the level line according to current condition elements.

- Concrete pier strenghtening works

- Mechanical or manual excavation in cascades with shoring, from the existing ground level to the level necessary for the planned pier reinforcement and the widening of the footing on abutment S6;

- Removal of damaged and unbound parts of concrete surfaces of the piers;

- Marking the layout of the holes for the installation of the vertical and horizontal reinforcement for the bonding of the old and the new concrete and the widening of the footing;

- Drilling holes which are 80cm deep, with diameter of 22mm at an angle of 15° from the horizontal axis for the installation of the reinforcement for the widening of the footing;

- Drilling holes which are 100(80)cm deep, with diameter of 28(22)mm at an angle of 15° from the verical axis for the installation of the vertical reinforcement for the pier;

- Drilling holes which are 80cm deep, with diameter of 20mm at an angle of 15° from the horizontal axis for the installation of the horizontal reinforcement for the pier;

- Cleaning and degreasing prepared concrete surfaces;

- The production of SN primer coating for the bonding of the old and the new concrete;

- The installation of horizontal anchors 16mm for the widening of the footing  on pier S6 and watering them with expansion plaster;

- The installation of vertical anchors 22(16)mm and watering them with expansion plaster for the reinforcement of the pier;

- The installation of horizontal anchors 14mm for the reinforcement of pier S6 and watering them with expansion plaster;

- The installation of the horizontal and vertical reinforcement for reinforced piers alongside welding with the existing reinforcement, wherever applicable;

- Making a layer of concrete C25/30 of the necessary thickness;

- Making protective bituminous coatings for the protection of concrete surfaces located below the ground level;

- Making a protective cement-based coating to protect concrete surfaces above the terrain level;

- Backfilling the reinforced parts of the piers.

The above mentioned works are executed in dry conditions.

- Repair works on concrete pier surfaces

Repair works on concrete pier surfaces including crack injection works:

- Identifying and marking cracks larger or equal to 0.2mm;

- Cleaning the marked concrete surfaces;

- Sealing cracks with appropriate epoxy gel;

- Injecting cracks with epoxy resin;

- Cleaning the treated concrete surfaces;

- All works are performed in line with the relevant Technical Specifications.

Repair works on concrete pier surfaces with cover degradation:

- Identifying and marking the surfaces to be repaired;

- Removing damaged and unbound parts of concrete pier surfaces;

- Cleaning the exposed and corroded reinforcement;

- Cleaning and degreasing the prepared concrete surfaces;

- Applying repair mortar;

- Treating repaired surfaces so that they are aligned with the surface of the existing concrete;

- Curing repair mortars/concretes;

- All works are performed in line with the relevant Technical Specifications.

The preparation of protective cement-based coating for the protection of concrete surfaces (vertical pier surfaces, bearing beams, pendulums, wing walls) in line with Technical Specifications.

The above mentioned works are executed in dry conditions.

- Conclusion

- The prerequisite for quality execution of damp-proofing works, gullies and expansion joints is to adhere to installation requirements and work carefully without any unreasonable demands to shorten the time required for such works.

- With good preparation and good management of work train traffic, sufficient number of construction machinery and experienced and qualified workforce, it is possible to repair damp-proofing on the deck at 15m of the bridge under one-day continuous closure (24h) and go to low-speed traffic afterwards.

This applies to the Vrbnica-Podgorica railway section for the period May-September and the Podgorica-Bar section for the period April-October.

- The Contractor shall, as per design recommendations and his technical and technological capacities determine which period of railway closures are necessary and submit a timely request to ŽICG for railway closures longer than the approved single-day closures.

- To the extent possible, traffic closures should be employed simultaneously on two or more bridges which are at the same inter-station distance.

- In bridges with spans over 15m, repeated use of long closures is necessary, although it is acceptable to employ such closures with breaks in between closures that are not longer than 2 days.

### List of base documents used to draft technical documentation

For the purpose of Technical Document drafting, the following designs and investigations were used:

- The original design - Main design of viaduct at km 83+168 of railway line Gostun - Titograd (developed by Institute for designing ZJŽ Belgrade, 1971.),

- Bridge condition report: Bridge in km. 366+293.45 of railway line Vrbnica - Bar (developed by Pro-inženjering, 2015.),

- Geotechnical elaborate (2018.),

- Geodetic survey (2018.)

### List of applied regulations

LAWS AND RULEBOOKS REGARDING THE CONTENTS OF THE TECHNICAL DOCUMENTATION

- Railway law. Official Gazette of RMN, N° 27/2013;

- Law on safety, organization and efficiency of rail transport of Montenegro of 27/12/2013, in force since January 2014;

- Law on spatial development and construction of structures. Official Gazette of RMN, N°51/08, 40/10, 34/11, 47/11, 35713, 39/13;

- Law on construction products N° 18/2014;

- Law on geological researches. Official Gazette of RMN, N° 28/93, 27/94, 42/94, 26/07;

- Law on occupational safety. Official Gazette of RMN, N° 34/2014;

- The Rulebook on content and production of technical documentation - Official Gazette of RMN, N° 23/14, 32/15, 75/15;

RULEBOOKS FOR THE DESIGN

- (316) Rulebook on technical standards for determining the size of the load and categorization of railway bridges, culverts and other structures on railway lines. Edition 1992

EN STANDARDS

- MEST EN 1990:2013-Eurocode - Basis of structural design.

- MEST EN 1990:2013/NA:2013- Eurocode - Basis of structural design - National Annex.

- MEST EN 1991-1-1:2017/NA:2017- Eurocode 1: Actions on structures - Part 1-1: General actions - Densities, self-weight, imposed loads for buildings - National Annex

- MEST EN 1991-1-3:2017-Eurocode 1 - Actions on structures - Part 1-3: General actions - Snow loads.

- MEST EN 1991-1-3:2017/NA:2017- Eurocode 1 - Actions on structures - Part 1-3: General actions - Snow loads - National Annex

- MEST EN 1991-1-4:2016-Eurocode 1: Actions on structures - Part 1-4: General actions - Wind actions.

- MEST EN 1991-1-4:2016/NA:2016- Eurocode 1: Actions on structures - Part 1-4: General actions - Wind actions - National Annex.

- MEST EN 1991-1-5:2017/NA:2017- Eurocode 1: Actions on structures - Part 1-5: General actions - Thermal actions - National Annex

- MEST EN 1992-1-1:2017/NA:2017 -  Eurocode 2: Design of concrete structures - Part 1-1: General rules and rules for buildings - National Annex

- MEST EN 1997-1:2017- Eurocode 7: Geotechnical design - Part 1: General rules - National Annex;

- MEST EN 1997-1:2017- Geotechnical design - Part 1: General rules;

- MEST EN 1998-1:2015 - Eurocode 8: Design of structures for earthquake resistance - Part 1: General rules, seismic actions and rules for buildings.

- MEST EN 1998-1:2015/NA:2015 - Eurocode 8: Design of structures for earthquake resistance - Part 1: General rules, seismic actions and rules for buildings - National Annex

- SRPS EN 1991-1-5:2012 - Eurocode 1: Actions on structures - Part 1-5

- SRPS EN 1991-1-1:2012 - Eurocode 1: Actions on structures -Part 1-1

- SRPS EN 1991-1-3:2012 - Eurocode 1: Actions on structures -Part 1-3

- SRPS EN 1992-1-1:2015 - Eurocode 2: Design of concrete structures: Part 1-1

- SRPS EN 1997-1:2004 - Eurocode 7: Geotechnical design - Part 1

- HRN EN 1991-1-7:2012 - Eurocode 1: Actions on structures - Part 1-7

- HRN EN 1991-1-7:2012/Cor.1:2015 - Eurocode 1: Action on structures -Part 1-7

- HRN EN 1991-1-7:2012/A1:2015 - Eurocode 1: Action on structures -Part 1-7

- HRN EN 1991-2:2012 - Eurocode 1: Action on structures -Part 2

- HRN EN 1992-2:2013 - Eurocode 2: Design of concrete structures -Part 2

- HRN EN 1998-2:2011 - Eurocode 8: Design of structures for earthquake resistance - Part 2

- HRN EN 1998-3:2011 - Eurokod 8: Design of structures for earthquake resistance -Part 3

- HRN EN 1998-3:2011/Cor.1:2014 - Eurokod 8: Design of structures for earthquake resistance -Part 3

- HRN EN 1998-5:2011 - Eurokod 8: Design of structures for earthquake resistace - Part 5

## 2.2.	Technical description - superstructure

### General

Within the Main Design for the rehabilitation of the bridge no. 70, works on the track superstructure in the function of bridge rehabilitation were planned. Technical documents for the bridge no. 70 deal with track section 86.00m long, which includes the total bridge length and 10.00m before and after the bridge.

Based on the Conclusion of the Inspection Report on the bridge no. 70 superstructure condition, and establishing the current condition in the drafting period of the relevant technical documents, works to be done on the bridge track superstructure were defined. The railway tracks on the bridge are geometrically correct , in transition curves of the opposite directions, with the elements of R=350m L=100m and R=400m L=90m, and are welded as CWR.

Measures were adopted to be taken on the bridge track superstructure with the drafting of Priced BoQ, Technical Specifications, Superstructure Static Calculation with the calculation of bridge-track interactions, with schedules no. 1-3 (Listings) and graphic schedule – Calculation model of bridge-track interactions with the results for summer and winter temperatures (axial forces in the rail in summer and winter and the displacement diagram for summer and winter)  and the Solution on bridge track development with the graphic schedule – Plan for track development on the bridge.

Total value of superstructure works for the bridge no. 70 amounts to 68,351.44€.

### Required measures and sequence of works on superstructure when executing bridge rehabilitation works

For the execution of rehabilitation works (placing new damp-proofing under the ballast prism, installation of new gullies and expansion joints) the following works on the superstructure were planned, where the bridge length and the length on the embankment of 10.00 m from abutments, before and after the bridge are taken into account:

- arresting CWR before and after the bridge with the installation of rail anchors;

- disassembly of the existing track in segments up to 22.5m with the track and check rail removal, whereby it is necessary to cut the existing track and drill rails for the connections at the joint, perform any disassembly of fasteners and sleepers as per technology of track and check rail removal;

- removal of the current crushed stone ballast all the way to the damp-proofing protective layer;

- installation of the track, the existing check rail and a new check rail on the opposite side, in segments up to 22.5m with new crushed stone, new sleepers and new fasteners, with the fitting of tie plates for running rails with a trimmed shorter point by 4 mm at length as per Plan for track development on the bridge with a fitting of connectors and the necessary fasteners for joining running rails and check rails;

- regulating the track by direction and the level line as per elements of current condition;

- disassembly of rail joints on running rails before welding;

- welding the track as CWR;

- installation of rail anchors in the track area before and after abutments, as well as devices against lateral track movement on the bridge and after the bridge in accordance with this design, that is, in line with the Plan for track development;

- final track regulation.

Bridge superstructure works on bridge rehabilitation, particularly the disassembly and the assembly of the existing track are executed under traffic closure while low-speed traffic scheme should be employed on the bridge section during rehabilitation works.

### Static calculation - summary

The basic elements of the superstructure used in the calculation of track stresses and stability are as follows:

- 49 E1 rails, grade R260 (900A)

- wooden sleepers L=260cm, at axial spacing of 60cm

- "K"-type fasteners.

The RC bridge structure is a continuous beam structure with five spans of L=2×12.0+18.0+2×12.0=66.0m.  The bridge extends over a dry valley, and the terrain is steeply sloped from left to right. Pier S6 has a fixed support, while the mobility of the system is achieved on the abutment S1 by a RC pendulum. The railway tracks in transition curves of the opposite directions, with the elements of R=350m L=100m and R=400m L=90m, and are welded as CWR.

With characteristics of all rods in the calculation model known, the calculation of stress resulting from temperature changes in summer and winter conditions was made. The calculation was made using STAAD software. The obtained results are in line with the starting assumptions of model operation. All results are given in chapter 3.2 Superstructure static calculation, based on which the Plan for track development was made.

Characteristic results

a) The greatest normal forces in CWR occur in the area of a bearing (bar 45) and have the following values:

in summertime Ns =  672.89 kN

in wintertime Nw = -882.14kN

b) The reactions of fixed supports on the bridge are as follows:

in summertime:  R52 =-136.57kN,

in wintertime:	R52 =240.27kN,

(they refer to pier halves).

c) The inspection of track stability against buckling in the curve with a radius R=350m and R= 400m on the bridge in extreme summer temperatures employing the Mischenko energy model was made.

Calculation results are shown in chapter 3.2.4, where values of critical lateral resistances were obtained as follows: for the curve R=350m, q= 121.61N/cm<160 N/cm, for which it is necessary to install sleeper anchors on every sleeper and for the curve R=400m, qkr=104.18Ncm<106N/cm for which it is necessary to install sleeper anchors on every third sleeper.

d) Inspection of crack size on rail cracking in winter was made on the location of highest tensile strength (bar 45). The following displacement values were obtained for disrupted rail sections: u10 = -1.25cm; u12 = 3.33 cm.

On rail cracking, the crack size shall be 1.25+3.33=4.58 < 10cm, i.e. traffic safety shall not be at risk in case of rail cracking.

e) The stress check in rail foot was done for stress UIC - 71 scheme 49 E1 rails, grade R260 (900A) and the speed of 80 km/h. The obtained stress was 11.92 kN/cm2. When the stress resulting from the highest axial temperature force and residual stress obtained from rail steel rolling is added to this, the total rail stress is as follows:

 = 11.92+14.01+6=31.93 kN/cm2   < 46 kN/cm2

meaning there is enough safety to withstand all other, secondary effects that have not been considered in the calculation.

### Track development

Based on the performed analysis and the obtained calculation results, the track should be developed in the following way meeting the following requirements:

- The substructure before and after the bridge must be completely and appropriately executed (cross-section, drainage, stabilization)

- Crushed stone ballast before, after and on the bridge must be uncontaminated, having the adequate quality and grain size distribution, with a ballast prism intended for CWR. On sections where rail anchors are placed, additional tamping of ballast prism should be performed.

- Before welding, the track (on the bridge and outside the bridge) must be completely regulated by direction and the level line as per the elements of the current condition.

- The calculation was made under the condition that both the railway line and the track on the bridge are welded as CWR.

- The required temperature for CWR tempering is +230C ± 30C.

- 74 pieces of rail anchors are fitted at about 50m of the track before and after the bridge starting from the last sleeper in the ballast before (or the first one after) the bridge.

- In order to decrease the impact of CWR on bridge piers, longitudinal resistance at the length of 16.20m (on ¼ of the entire bridge span) from the movable support S1 to fixed support S6 was decreased, as per Track development plan. On the field, this decrease in the resistance is achieved by fitting shortened tie plates (cutting the shorter point of the plate by 4mm) at 16.20 m (27 sleepers) from S1 to S6.

- Based on performed checks of track stability against buckling in the curve with the radius R=350m and R=400m, it is necessary to install sleeper anchors on the bridge as follows:

Devices against lateral track movement (47 devices in total), are installed  10.8m before the abutment S1 and 7.2m after it (from bar 44 to bar 47).

Devices against lateral track movement (5 devices in total), are installed  5.74m from bar 48 on every second sleeper, and from that point on, up to the end of the bridge, devices against lateral track movement are installed on every third sleeper (30 devices in total).

The existing devices against lateral track movement are installed on every second sleeper in the curve before the bridge up to 10.8m before the abutment S1, and on every third sleeper after the bridge, in accordance with the Track development plan; there are 33 devices before the bridge (from km 366+210.78 to km 366+249.98) and another 23 devices after it (from km 366+336.78 to km 366+376.78). The before mentioned existing devices against lateral track movement are not included in the Priced Bill of Quantities.

- Permanent labels for monitoring CWR longitudinal and transversal displacement (in the bridge area) should be put in the following places:

- at the first sleeper after the bridge

- at the first sleeper before the bridge.

Labels are buried at both sides of the track on stable soil.

Labels are placed before completing CWR formation, and are calibrated immediately after CWR tempering in the presence of the Supervisor.

Chainage labels can be used as permanent labels for CWR monitoring, provided they are in the immediate vicinity (within 3 m) of the places where labels are placed. Permanent labels are not included in the Priced Bill of Quantities.

- The existing check rail shall be used, and a new check rail shall be installed on the opposite side, and these rails shall be interconnected with an oak key, whereby the new fasteners are installed on the sleeper connection with the running rails and check rails, which connection is made at each sleeper, in addition to new fasteners for bridge ends and new additional fasteners for check rail joints.

- All the necessary track maintenance works should be done on time, in a quality manner and within the allowed temperature ranges. It is also necessary to ensure enhanced supervision on the section at extreme rail temperatures (below  -10°C and over +35°C).