2. TEXTUAL DOCUMENTATION

________________________________________________

2.1. Technical description – bridge structure

________________________________________________

2.1.1. General project data

| EMPLOYER: | European Investment Bank |
| --- | --- |
| BENEFICIARY: | Željeznička infrastruktura Crne Gore AD Podgorica |
| STRUCTURE: | Bridge no. 34 at km 326+449.50 |
| PROJECT: | MAIN DESIGN FOR REHABILITATION OF BRIDGE no. 34 at km 326+449.50 |
| SECTION: | Vrbnica – Bar |
| CHAINAGE: | 326+449.50 |
| PROJECT STAGE: | Main design |
| NAME AND DESIGNATION OF PROJECT SECTION: | VOLUME 2.1 – STRUCTURAL DESIGN Bridge no. 34 at km 326+449.50 |
| PROJECT ORGANIZATION RESPONSIBLE FOR DRAFTING BRIDGE STRUCTURAL DESIGN: | Cestra d.o.o. Beograd Makenzijeva 57, 11118 Beograd |

2.1.2. 	Description of route location and alignment

The bridge structure is placed in a hill cut-and-fill. The terrain under the bridge is unstable and erosion-prone, as can be seen, particularly in the area of bridge piers where the foundations are in the shallow ground.

The bridge extends over a seasonal stream running from the hill. This is an unregulated watercourse with traces of deposits. The left side of the bridge is too close to the ground (the lowest measured height is 0.5m) so the left side of the structure and possibly the track are at risk of becoming covered.

The drainage channel extends from the left side of the bridge, which has not been properly solved in the pier area. The water from the channel drops to the ground in the pier area causing erosion.

The track on the bridge structure is in a transitional curve and is welded as CWR.

In the area of the structure there is space for construction site capacities.

2.1.3. Attachment 1 – Bridge layout plan from the ŽICG archives

2.1.4. Attachment 2 – Technical Report from the original design

2.1.5. Attachment 3 – Assessment of bridge condition with proposed measures (Bridge inspection report from 2014)

Taken from „The report on the state of the bridge: the bridge at km 324+983,98 the railway line Vrbnica – Bar” from 2014.

General observation about the state of the bridge:

Measuring the dimensions of the structural elements and comparing with the dimensions given in layout sketches from " Report about the current state of bridges on the Vrbnica-Bar railway line" it was observed:

- dimensions of the bridge are not in compliance with the dimensions from the "Report about the current state of bridges on the Vrbnica-Bar railway line". The exact dimensions are shown in Appendix 1 of this Report,

- dimensions of abutments wingwalls were not included in the layout sketch of the bridge. They were measured on the site and shown in Appendix 1 of this Report,

- sidewalk cantilevers and their’s details are not in compliance with layout sketch. The exact dimensions are shown in Appendix 1 of this Report,

- after removing the track ballast and measuring of all necessary dimensions,  thickness of the RC slab was determined indirectly and it was found to be 67 cm (with a protective coating and waterproofing).

Railway track on the bridge is in good condition. After removing the track ballast in the sleeper zone,thickness of the track ballast below the lower edge of sleeper was found to be 27 cm (in the axis of the railway track).

Damages that could reduce the capacity and stability of the bridge structure were not found during the detailed visual inspection of the bridge.

However,prominent scour and erosion observed in the right (lower) zone of abutments with it’s progression can result in unstability of the abutments as well as the unstability of the whole bridge structure.

Also,very poor condition of the connection between steel column for electric power and massive retaining wall behind the abutment S1 (abutment towards Vrbnica) dangerously jeopardize the stability of the steel column for electric power as well as the safety of railway traffic in the zone of the bridge.

The observed damages and deficiencies of the structure which negatively affect the durability and functionality of the structure are the following:

- increased scour and erosion in the zone of abutments foundations,on the right-lower side,which may,with it’s progression,result in total erosion of the ground beneath foundations and dangerously threaten the stability of the abutments,as well as the whole bridge structure,

- very poor condition of the connection between steel column for electric power and massive retaining wall behind the abutment S1 (abutment towards Vrbnica) dangerously jeopardize the stability of the steel column for electric power as well as the safety of railway traffic in the zone of the bridge,

- expansion joints are out of function (or even does not exists), and that results in inflow of the water from the railway track to the backwalls, cap beams, abutmens and structure in the zone of the abutmens,

- the absence of the drip edges on the sidewalk cantilievers that leads to the inflow of the water on the bridge structure especially on the right side,

- lack of drainage of the installation channels which results in sporadic water penetration through concrete of sidewalk cantilevers.

## Proposed measures from the aspect of the bridge structure durability and functionality

- stop scour and erosion and make the ground in the zone of abutments stable. Before execution of works on this position – stabilizing the ground, it is necessary to perform a detail inspection of ground and observed phenomena by specialized experts for ground erosion who will give their opinion and solution of this problem with technical details,

- rehabilitation of concrete in the zone of steel column for electric power connection to retaining wall (behind the abutment S1 towards Vrbnica) and after that fasten the bolts in connection,

- replacement of the damaged watertight expansion joints,

- making of the drip edges on the sidewalk cantilevers,

- provide drainage of the installation channels,

- installation of the missing covers for concrete installation channel in the zone of installation transition.

Urgent measures on the bridge construction

Next measures, listed above in point Proposed measures, should be done urgently:

- organize a detailed visual inspection of the ground in the zone of abutments by specialized experts for ground erosion,

- rehabilitation of concrete in the zone of steel column for electric power connection to retaining wall (behind the abutment S1 towards Vrbnica) and after that fasten the bolts in connection.

All damages that need to be treated (listed above) doesn’t reduce capacity and stability of the bridge at this moment, however lack of rehabilitation of these damages and their progression may result in reduced capacity and stability of the bridge in the short term as well as in threatened functionality of electric power contact line in the zone of bridge.

2.1.6. Damage to the bridge and possible causes

A detailed summary of damage with possible causes was given in the previous project stage:

˝Report on bridge condition: bridge at km 326+449.50 on Vrbnica – Bar railway line˝ from 2015 prepared by Pro-Inženjering d.o.o. in Belgrade

A graphic representation of damage observed on bridge inspection is given in graphic documents of the design file in the drawing – Photograph of damage.

Based on the "Report on bridge condition bridge at km 326+449.50 on Vrbnica – Bar railway line˝ from 2015 the scope of damage was analysed alongside the methods for its rehabilitation. Scope of works and investment value were based on an inspection performed in 2015.

2.1.7. Description of bridge structure – current condition

The bridge structure is made of reinforced concrete, has a box cross-section and consists of a simply-supported beam with a span of L=9.00m. The track on the bridge structure is in a transitional curve and is welded as CWR. The bridge is in a cut-and-fill and it extends over a seasonal torrential stream.

The main girder is a box-type girder with constant thickness of 70 cm and the width of 440 cm having cantilevers on both sides 45 cm each where inspection walkways were made with cable ducts.

The main girder is fitted directly onto RC sleeper beam measuring 86x100cm, joint to it with anchors forming a fixed bearing. Given that anchors were used to connect the main girder on both piers to sleeper beams, both piers take equal part in the transfer of horizontal forces. Abutments are massive, made of plain concrete and founded on footings that were founded in a cascade fashion in the rock forming a filling in open pits, thus tying the structure to the rock mass. After RC abutments there are parallel wing walls behind which massive retaining walls are built.

From the upper side of the bridge deck, damp-proofing was placed with a concrete cover.

There is no drainage system on the bridge structure.  Drainage on the bridge was handled using a double-sided slope of a slab placed longitudinally in the railway embankment.

The handrail on the footpath on the bridge structure was made of metal pipes with a welded circular cross-section. The handrail is 1100 mm high.

The following cables were laid on the bridge structure in the left and right concrete duct:

- main railway STKA cable

- optic cable G625 144V 6x24 -  9/125 in PE pipe Ø 40 + a spare PE pipe  Ø 40

There is no drainage in the cable ducts.

The contact line pole was placed in the right-side area of abutment S1. The pole is tied to a massive retaining wall.

For the reinforced concrete elements the round concrete steel of Č37 (240/360Mpa) grade was used.

Concrete grade used in structural elements:

Main girders - MB300 - which corresponds to concrete grade C25/30 according to EN 1992-1-1

Abutments - MB160 - which corresponds to concrete grade C12/15 according to EN 1992-1-1

Sleeper beams - MB220 - which corresponds to concrete grade C16/20 according to EN 1992-1-1

Footings - MB110 - which corresponds to concrete grade C8/10 according to EN 1992-1-1

Wing walls - MB220 - which corresponds to concrete grade C16/20 according to EN 1992-1-1

The Report on bridge condition from 2015 states the following:

- there is enhanced erosion in the area of abutment foundation on the right, the progression of which could lead to complete washing out of soil beneath the abutment foundation, which could seriously endanger abutment stability, and consequently the stability of the structure.

After obtaining the original project documents (with as-built drawings) it was determined that foundations in the executed structure are deeper than in the original variant, hence the terrain is much more favourable compared to the depth of foundation. Hence, the conclusion of the Report on bridge condition expressing doubt about stability of piers is not valid.

The concrete channel for water reception before the bridge is inclined from abutment S1 to Vrbnica and therefore there is no runoff of water into the area of abutment S1. The concrete channel for water reception after the bridge is inclined from Bar to pier S2, hence the water from the channel, the concrete (sloped) structure is directed to the middle of the span onto the earth slope.

Before the drafting of the Design for the rehabilitation of the present bridge began, the Designer had consultations with a water engineer regarding the influence of surface waters on the stability of the slope and the water engineer came up with the following conclusion:

"Bridge 34 does not extend over the torrential stream as specified in the description of the existing state, instead this is a riparian section between two minor catchments of 2.05ha, the power of which is insufficient to endanger the structure stability. The water from this subcatchment moves towards pier S2 approaching the railway in a parallel fashion from the direction of Bar with a minor concrete channel.  Therefore, it is not necessary to regulate the watercourse in the bridge area but rather to clean the existing channels and bring them into operating condition."

2.1.8. Static calculation - summary

2.1.9. Rehabilitation works

It is necessary to perform the following rehabilitation works on the bridge:

- controlled demolition of concrete in footpaths (while retaining the existing reinforcement) and dismantling of the existing handrail in the peripheral area in order to place a new cornice,

- dismantling and removal of present expansion joints,

- controlled demolition of concrete in RC bridge deck, parapet and walls of the cable ducts in order to install new expansion joints,

- replacing the present damp-proofing on the entire bridge,

- installation of new waterproof expansion joints on both abutments,

- drilling holes for anchor placement and cleaning the existing concrete with pressurized air, applying SN coat, placing anchors to connect the old concrete with the new concrete in RC parapets of the crushed stone ballast bed and grouting with non-shrinkage mortar,

- building RC parapets on both sides of the crushed stone ballast bed,

- placement of cable duct lids made of corrugated sheet metal in places where expansion joints are put,

- repairing cable ducts,

- repairing cantilever cornices on footpaths,

- placement of new handrails on footpaths,

- repair works on concrete surfaces and anti-corrosive measures on reinforcement in places where concrete surfaces are damaged,

- applying damp-proofing coat over tread areas of footpaths,

- applying protective concrete coating on all visible concrete surfaces after preparing the concrete surface using abrasive methods.

2.1.10. Technology of work execution

REHABILITATION WORKS EXECUTED ON THE TRACK OR IMMEDIATELY BY THE TRACK

Given that each structure has different conditions for the transport of personnel, equipment, machinery and materials, the following principles were adopted:

- The construction site is erected in the area of the structure in case:

- there is an asphalt road or a good macadam road;

- there is free space for construction site capacities.

- The construction site is erected in the area of the nearest railway station and in that case the following applies:

- transport of personnel and material to the bridge is carried out using railway rolling stock during the whole time of works as per established procedures and possible time slots;

- make sure to provide a building for the security personnel, a building for workers' accommodation and the tool warehouse at the bridge location.

The following was adopted for bridge no. 34:

- there is no access road to the bridge;

- the central construction site should be placed in the Mojkovac station at km 321+400 as there is a road for truck and other vehicles leading there;

- the haulage of equipment, machinery and material from the Mojkovac station to the bridge is done with railway rolling stock;

- transport of personnel is possible via the asphalt road 300m from the bridge location.

- General

Here is a detailed description of the technology for the items of rehabilitation works executed on the track or immediately by the track. Other items of works are executed independently of railway traffic and are not in the high voltage area. The items of works for which the execution technology is described in detail are the following:

- Placement of new damp-proofing under ballast prism

Removal of the cover layer (fine-grained concrete) and removal of existing damp-proofing, sand blasting the concrete surface, applying repair mortar, producing new sprayed damp-proofing with a cover made of prefabricated sheets of extruded polystyrene.

- Placement of new bridge expansions

Disassembly of existing expansions, treatment of concrete surfaces, fitting new expansions

- Repairing cable ducts

Removing sand from the ducts, local displacement of SS and TT cables, sandblasting concrete surfaces, applying repair mortar, fitting tubes for water infiltration, producing the sloping layer and sand blasting the damp-proofing layer.

- Repairing cantilever cornices on footpaths

Machine removal of the top and lateral surface of concrete cornices, sandblasting the concrete and the rebar, producing new section of the cornice with a drip cap.

- Placement of new handrail on footpaths

Removal of existing handrail on footpaths, the placement of new handrail on previously repaired cornices.

- Building RC parapet of the crushed stone ballast bed

- material haulage, making shoring for the stability of the crushed stone ballast, temporary removing and reinstalling a part of the ballast prism, concrete haulage and parapet concrete placement and dismantling of shoring,

- making formwork, drilling holes in concrete, fitting anchors with exmal grouting, fitting reinforcement, concrete curing and formwork dismantlement.

It is essential to make new damp-proofing and new expansions in a high-quality manner so as to eliminate harmful effect of water on the structure long term. Placement of damp-proofing and the repair mortar coating must be done adhering to temperature requirements, humidity and hardening times. It is also important to avoid frequent resumption of works when damp-proofing as much as possible.

The above-mentioned items of works are executed under special railway traffic management schemes as follows:

- item 1 is executed under traffic closure and voltage shutdown in the maximum period of time;

- item 2 is executed under “railway closure for construction works” scheme in day-time slots (e.g. 11:30 to 16:30) with or without power shutdown;

- items 3, 4, 5 and 6 are executed under low-speed traffic scheme (30 km/h) without power shutdown.

Works defined under items 2 through 6 may be executed under railway traffic scheme a) or b) if this does not interfere with the works defined under item 1 in the Description of works.

- Track works (the bridge and the bridge area)

For the execution of works under items 1 and 2, it is necessary to remove the rails, sleepers and crushed stone ballast all the way to the damp-proofing layer.

Here is the description of items of track works that need to be executed.

- Works to be executed before traffic closure:

- assembly and disassembly of rail anchors in the track area after abutments;

- assembly and disassembly of sleeper anchors in the track area after abutments;

- cutting rails in the embankment at 10m from abutments and at every 22.5m on the bridge with rail drilling and fitting of rail connectors.

- Works executed under traffic closure and power shutdown

- track disassembly (rails, check rails, sleepers, ballast prism) of one 15m span;

- assembling the track using existing rails, the new crushed stone ballast, new wooden sleepers and new rail fasteners.

- Works executed after construction works in the track area

- welding rails as CWR, placement of check rails;

- regulating the track by direction and the level line according to current condition elements

- Conclusion

- The prerequisite for quality execution of damp-proofing works, gullies and expansion joints is to adhere to installation requirements and work carefully without any unreasonable demands to shorten the time required for such works.

- With good preparation and good management of work train traffic, sufficient number of pieces of construction machinery and experienced and qualified workforce, it is possible to repair damp-proofing on the deck on 15m of the bridge under one-day continuous closure (24h) and go to low-speed traffic afterwards.

This applies to the Vrbnica-Podgorica railway section for the period May-September and the Podgorica-Bar section for the period April-October.

- The Contractor shall, as per design recommendations and his technical and technological capacities determine which periods of railway closures are necessary and submit a timely request to ŽICG for railway closures longer than those approved on a daily basis.

- To the extent possible, traffic closures should be employed simultaneously on two or more bridges which are at the same inter-station distance.

- In bridges with spans over 15m, repeated use of long closures is necessary, although it is acceptable to employ such closures with breaks in between closures that are not longer than 2 days.

### 2.1.11.  The list of templates used for the creation of technical documentation

The design engineer used the following documents for the creation of the rehabilitation project:

- The original design - Main design of  RC concrete bridge at km 42+880 of railway line Gostun - Titograd  (developed by Institute for designing ZJŽ Belgrade, 1966.);

- Geological study about investigation works for the viaduct at km 42+882 of railway line Gostun - Titograd (developed by Institute for designing ZJŽ Belgrade, 1965.);

- Bridge condition report: Bridge in km. 326+449,50 of railway line Vrbnica - Bar (developed by Pro-inženjering, 2015.);

- Geotechnical elaborate (2018.);

- Geodetic survey (2018.).

### 2.1.12.   List of applied regulations

LAWS AND RULEBOOKS REGARDING THE CONTENTS OF THE TECHNICAL DOCUMENTATION

- Railway law. Official Gazette of RMN, N° 27/2013;

- Law on safety, organization and efficiency of rail transport of Montenegro of 27/12/2013, in force since January 2014;

- Law on spatial development and construction of structures. Official Gazette of RMN, N°51/08, 40/10, 34/11, 47/11, 35713, 39/13;

- Law on construction products N° 18/2014;

- Law on geological researches. Official Gazette of RMN, N° 28/93, 27/94, 42/94, 26/07;

- Law on occupational safety. Official Gazette of RMN, N° 34/2014;

- The Rulebook on content and production of technical documentation - Official Gazette of RMN, N° 23/14, 32/15, 75/15;

RULEBOOKS FOR THE DESIGN

- (316) Rulebook on technical standards for determining the size of the load and categorization of railway bridges, culverts and other structures on railway lines. Edition 1992

EN STANDARDS

- MEST EN 1990:2013-Eurocode - Basis of structural design.

- MEST EN 1990:2013/NA:2013- Eurocode - Basis of structural design - National Annex.

- MEST EN 1991-1-1:2017/NA:2017- Eurocode 1: Actions on structures - Part 1-1: General actions - Densities, self-weight, imposed loads for buildings - National Annex

- MEST EN 1991-1-3:2017-Eurocode 1 - Actions on structures - Part 1-3: General actions - Snow loads.

- MEST EN 1991-1-3:2017/NA:2017- Eurocode 1 - Actions on structures - Part 1-3: General actions - Snow loads - National Annex

- MEST EN 1991-1-4:2016-Eurocode 1: Actions on structures - Part 1-4: General actions - Wind actions.

- MEST EN 1991-1-4:2016/NA:2016- Eurocode 1: Actions on structures - Part 1-4: General actions - Wind actions - National Annex.

- MEST EN 1991-1-5:2017/NA:2017- Eurocode 1: Actions on structures - Part 1-5: General actions - Thermal actions - National Annex

- MEST EN 1992-1-1:2017/NA:2017 -  Eurocode 2: Design of concrete structures - Part 1-1: General rules and rules for buildings - National Annex

- MEST EN 1997-1:2017- Eurocode 7: Geotechnical design - Part 1: General rules - National Annex;

- MEST EN 1997-1:2017- Geotechnical design - Part 1: General rules;

- MEST EN 1998-1:2015 - Eurocode 8: Design of structures for earthquake resistance - Part 1: General rules, seismic actions and rules for buildings.

- MEST EN 1998-1:2015/NA:2015 - Eurocode 8: Design of structures for earthquake resistance - Part 1: General rules, seismic actions and rules for buildings - National Annex

- SRPS EN 1991-1-5:2012 - Eurocode 1: Actions on structures - Part 1-5

- SRPS EN 1991-1-1:2012 - Eurocode 1: Actions on structures -Part 1-1

- SRPS EN 1991-1-3:2012 - Eurocode 1: Actions on structures -Part 1-3

- SRPS EN 1992-1-1:2015 - Eurocode 2: Design of concrete structures: Part 1-1

- SRPS EN 1997-1:2004 - Eurocode 7: Geotechnical design - Part 1

- HRN EN 1991-1-7:2012 - Eurocode 1: Actions on structures - Part 1-7

- HRN EN 1991-1-7:2012/Cor.1:2015 - Eurocode 1: Action on structures -Part 1-7

- HRN EN 1991-1-7:2012/A1:2015 - Eurocode 1: Action on structures -Part 1-7

- HRN EN 1991-2:2012 - Eurocode 1: Action on structures -Part 2

- HRN EN 1992-2:2013 - Eurocode 2: Design of concrete structures -Part 2

- HRN EN 1998-2:2011 - Eurocode 8: Design of structures for earthquake resistance - Part 2

- HRN EN 1998-3:2011 - Eurokod 8: Design of structures for earthquake resistance -Part 3

- HRN EN 1998-3:2011/Cor.1:2014 - Eurokod 8: Design of structures for earthquake resistance -Part 3

- HRN EN 1998-5:2011 - Eurokod 8: Design of structures for earthquake resistace - Part 5

2.2. Technical description – superstructure

________________________________________________

2.2	 TECHNICAL DESCRIPTION - SUPERSTRUCTURE

2.2.1	GENERAL

Within the Main Design for the rehabilitation of bridge no. 34 works on the track superstructure are planned, which are to serve as bridge rehabilitation. Technical documents for bridge no. 34 analysed a track section 29m long, which includes the entire bridge length and 10m before and after the bridge.

Based on the Conclusion of the Inspection Report on the condition of bridge 34 superstructure and establishing the current condition in the drafting period of the relevant technical documents, works to be done on the bridge track superstructure were defined. The track on the bridge structure is in a curve (R=300 L=100) placed on a part of a transition curve and is welded as CWR.

The Measures regarding the superstructure with the drafting of the Priced BoQ, Technical Specifications, Superstructure Static Calculation and the Plan for track development were defined.

Total value of superstructure works for bridge no. 34 is €24,415.16

2.2.2	MEASURES REGARDING THE SUPERSTRUCTURE

For the execution of rehabilitation works (placing new damp-proofing under the ballast prism, installation of new gullies and expansion joints) the following works on the superstructure were planned, where the bridge length and the length on the embankment of 10 m from abutments, before and after the bridge are taken into account:

- arresting CWR before and after the bridge with the installation of rail anchors;

- disassembly of the existing track in segments up to 22.5m accompanied by the removal of the track, whereby it is necessary to cut the existing track and drill rails for the connections at the joint, any disassembly of fasteners and sleepers as per track technology used for the removal of the track;

- removal of current crushed stone ballast all the way to the damp-proofing protective layer;

- installation of the track in segments up to 22.5m with new crushed stone, new sleepers and new fasteners, with the fitting of tie plates for running rails with a trimmed shorter point by 4 mm along the entire bridge length with the fitting of connectors and the necessary fasteners for joining running rails;

- regulating the track by direction and the level line according to current condition elements;

- disassembly of rail joints on running rails before welding;

- welding the track as CWR;

- disassembly of rail anchors, placed in order to arrest CWR before and after the bridge;

- installation of devices against lateral track movement on the bridge;

- final track regulation.

Bridge superstructure works for bridge rehabilitation, particularly the disassembly and the assembly of the existing track, are executed under traffic closure while low-speed traffic scheme should be employed on the bridge section during rehabilitation works.

2.2.3	STATIC CALCULATION-SUMMARY

The basic elements of the superstructure used in the calculation of track stresses and stability are as follows:

- 49 E1 rails, grade R260 (900A)

- wooden sleepers L=260cm, at axial spacing of 60cm

- "K"-type fasteners.

The bridge structure is made of reinforced concrete, has a box cross-section and consists of a simply-supported beam with a span of L=9.00 m. The main girder is fitted directly onto RC piers with parallel wing walls behind which massive retaining walls are built. The track on the bridge structure is in a curve (R=300 L=100) placed on a part of a transition curve and is welded as CWR.

Having in mind the provisions of the Rulebook on railway superstructure maintenance (“Official Journal of Montenegro, no.  42/2016 of 11 July 2016), the applicable EN standards and UIC publications, and the view that there is no relative displacement between the bridge and the continuous welded track (Study of various methods adopted by world railways to continue LWR over bridges), the calculation was made accounting for the effect of temperature changes in the rail without the effect of bridges on the rail (as on an open railway, i.e. the track was observed on the embankment).

Characteristic results

a) For a continuous welded track made of 49E1 the maximum compression strength is:

N = - 661.50kN

b) For a continuous welded track made of 49E1 the maximum tensile strength is:

N = 817.14kN

c) The inspection of track stability against buckling in the curve with a radius R=300m on the bridge in extreme summer temperatures employing the Mischenko energy model was made.

They obtained critical lateral resistance values of q = 112.32 N/cm < 118 N/cm for which it is necessary to fit sleeper anchors at every second sleeper.

d) Inspection of crack size on rail cracking in winter determined that on rail cracking, the crack size shall be 2.52cm<10cm, i.e. traffic safety shall not be at risk in case of rail cracking.

e) In order to reduce the impact of CWR on bridge piers, longitudinal resistances were reduced on the total bridge length of 9m. On the ground this reduction in resistances is achieved by fitting trimmed tie plates (by trimming a  shorter point by 4 mm) in the distance of 9m (15 sleepers).

2.2.4	SOLUTIONS FOR TRACK DEVELOPMENT

Based on the performed analysis and the obtained calculation results, the track should be developed in the following way meeting the following requirements:

- The substructure before and after the bridge must be completely and properly executed (cross-section, drainage, stabilization).

- Crushed stone ballast before, after and on the bridge must be uncontaminated, having the recommended quality and grain size distribution, with a ballast prism intended for CWR. On sections where rail anchors are placed, additional tamping of ballast prism should be performed.

- Before welding, the track (on the bridge and outside the bridge) must be completely regulated by direction and the level line as per the elements of the current condition.

- The calculation was made under the condition that both the railway line and the track on the bridge are welded as CWR.

- The required temperature for CWR tempering is +230C ± 30C.

- Before CWR arrest, 74 pieces of rail anchors are fitted on about 50m of the track before and after the bridge, which shall be dismantled after final welding.

- In order to reduce the impact of CWR on bridge piers, longitudinal resistances were reduced on the total bridge length of 9m. On the ground this reduction in resistances is achieved by fitting trimmed tie plates (by trimming the shorter point by 4 mm) in the distance of 9m                (15 sleepers).

- Based on performed checks of track stability against buckling in the curve with the radius R=300m, it is necessary to install sleeper anchors at every second sleeper, as follows: 25 pieces of devices against lateral track movement from km 326+435.00 to km 326+464.00 in the curve (R=300 and L=100). Please note that it is necessary to check outside the bridge section if the

devices against lateral track movement were fitted, as follows: 34 pieces of devices against lateral track movement before the bridge (from km 326+395.00 to km 326+435.00 in the curve R=300 and L=100), and 24 pieces of devices against lateral track movement after the bridge on every other sleeper (from km 326+464.00 to km 326+492.54 in the curve R=300 and L=100) and 7 pieces on every third sleeper (from km 326+492.54 to km 326+504.00 in the curve R=500 and L=70). The devices against lateral track movement outside the bridge section mentioned above are not included in the Priced BoQ.

- Permanent labels for monitoring CWR longitudinal and transversal displacement (in the bridge area) should be put in the following places:

- at the first sleeper after the bridge

- at the first sleeper before the bridge.

Labels are buried on both sides of the track on stable soil.

Labels are placed before completing CWR formation, and are calibrated immediately after CWR tempering in the presence of the Supervisor.

Chainage labels can be used as permanent labels for CWR monitoring, provided they are in the immediate vicinity (within 3m) of the places where labels are placed. Permanent labels are not included in the Priced Bill of Quantities.

- All the necessary track maintenance works should be done on time, in a quality manner and within the allowed temperature ranges. It is also necessary to ensure enhanced supervision on the section at extreme rail temperatures (below  -10°C and over +35°C).

3. NUMERICAL DOCUMENTS

________________________________________________

3.1. Static calculation for bridge structure

________________________________________________

STATIC CALCULATION CONTENTS

    - INPUTS

      - Structure layout - main design (1996)

      - Data from the main design (1996)

      - Cross-section of the structure - newly designed (2017)

      - Material characteristics

        - Concrete characteristics

        - Reinforcement characteristics

        - Foundation soil characteristics

      - Partial safety factors related to materials

      - Calculation model

      - List of loads

        - Basic calculation (ULS and SLS)

        - Seismic calculation (ULS)

        - Forming combinations of loads

    - LOAD ANALYSIS

      - Permanent loads Gki,S

        - Dead load Gk,1

        - Permanent load on the bridge Gk,2

        - Earth pressure Gk,3

        - The effects of concrete creep and shrinkage S

      - Variable actions Q,T,F,s

        - Movable load from the train Qk,1

        - Requirements for static or dynamic calculation

        - Dynamic coefficients

        - "Empty wagons“ loading model qk,2

        - Imposed load on inspection walkways qfk

        - Centrifugal force Qtk

        - Lateral force Qsk

        - Forces arising from train breaking and train starting Qlak,qlak,Qlbk,qlbk

        - Temperature effects Tk

        - Wind loads Fwk

        - Snow loads sk

        - Effect of horizontal forces in CWR qcwr

      - Seismic forces AE

        - Seismic forces – bridge structure AEk

        - Seismic earth pressure AEd

      - Accidental actions Ad

        - Effects of  derailment on the bridgeQA1d,qA1d,qA2d

    - POSITIONS OF LOADS

    - PRINCIPLES OF LOAD COMBINATIONS

      - Summary of loads

      - Excerpt from EN1991-2 and EN1990

      - Ultimate limit state ULS

        - Combination of actions for permanent and transient calculation situations (basic combinations)

        - Combinations of actions for accidental calculation situations

        - Combinations of actions for seismic calculation situations

        - Combinations of actions for fatigue calculation (approximate method)

      - Serviceability limit state SLS

        - Combination of actions for irreversible serviceability limit state

        - Quasi-constant combination of actions for long terms actions and structure appearance

    - SERVICEABILITY LIMIT STATE (SLS)

      - Pavement structure sagging

      - Vertical deformations of the bridge pavement structure

        - Rotations in the ends of the bridge pavement structure

        - Vertical motion in the ends of the bridge pavement structure

      - Torsion of the bridge pavement structure

      - Longitudinal motion in the ends of the bridge pavement structure

        - Longitudinal motion in the ends of the bridge pavement structure due to LM71

        - Longitudinal motion in the ends of the bridge pavement structure due to breaking and starting forces

      - Transverse deformations of the bridge pavement structure

      - Lateral vibrations in the bridge pavement structure

      - Calculation of cracks in the RC bridge deck

      - Calculation for new expansion joints

        - Approving expansion joints

    - ULTIMATE LIMIT STATE (ULS)

      - Main girder - RC bridge deck

        - Static effects in the bridge deck

        - Diagrams of necessary reinforcement in the bridge deck

        - Existing reinforcement in the bridge deck

        - Checking shear capacity in the bridge deck

        - Fatigue calculation

        - Summary of obtained results and conclusion on the bridge deck bearing capacity

      - Pier 2 - design pier with fixed bearing

        - Checking normal stress in cross-section 1-1

        - Checking shear stress in cross-section 1-1

        - Checking normal stress in cross-section 2-2

        - Checking shear stress in cross-section 2-2

        - Checking stresses in footing joint

        - Summary of obtained results and conclusion on pier S2 bearing capacity

      - Wing walls

        - Static effects in wing walls

        - Diagrams of necessary reinforcement in wing walls

        - Existing reinforcement in wing walls

        - Summary of obtained results and conclusion on wing walls bearing capacity

      - Fixed support on abutment S1

        - Calculation for anchors joining the sleeper beam and the bridge deck

        - Summary of obtained results and conclusion on fixed bearing capacity

      - Cantilever of  (inspection) walkway

        - Cantilever load

        - Static effects in the cantilever

        - Combination of loads for cantilever design

        - Diagrams of necessary reinforcement in cantilever

        - Existing reinforcement in cantilever

        - Summary of obtained results and conclusion on cantilever bearing capacity

      - New handrail on (inspection) walkway

        - Calculation for handrail pole

        - Calculation for handrail rail

        - Calculation for the pole connection to frontal slab

        - Calculation for the anchors connected to frontal slab

    - SEISMIC CALCULATION ACCORDING TO EN1998 (ULS)

      - Longitudinal and transversal direction - X and Y direction

        - Calculation model for X and Y directions

        - Oscillation periods, participation of mass, seismic forces

      - Abutment S1 - design pier with fixed bearing

        - Checking normal stress in cross-section 1-1

        - Checking shear stress in cross-section 1-1

        - Checking normal stress in cross-section 2-2

        - Checking shear stress in cross-section 2-2

        - Checking stresses in footing joint

        - Summary of obtained results and conclusion on abutment S1 seismic bearing capacity

      - Fixed support on abutment S1

        - Calculation for anchors joining the sleeper beam and the bridge deck due to seismic actions

        - Summary of obtained results and conclusion on fixed support bearing capacity