#### Western Balkans Investment Framework Infrastructure Project Facility Technical Assistance 5 (IPF 5)

#### TA2015030 R0 IPA

Subproject: WB14-MNE-TRA-01

Reliability and Safety Improvement of “Bar-Vrbnica” Railway Line (SEETO Rail Route 4) – Phase 2                      Main Designs for Rehabilitation of 90 Concrete Bridges

#### Main Design for Rehabilitation of bridge No. 41 at km 332+539.76 of “Bar-Vrbnica” Railway

- Book 4 – Annexes

- Volume 4.1- Project impact on Environment

- March, 2018.

BOOK 4 – ANNEXES

VOLUME 4.1 – Project impact on Environment

Bridge no. 41 at km 332+539.76

________________________________________________

- CONTENT OF VOLUME 4.1

- 1.	GENERAL DOCUMENTATION

- 1.1.	Excerpt from the Central Registry for the company which made part of technical documentation

- 1.2.	Company license for preparing the part of the technical documentation

- 1.3.	Decision on appointment of Responsable Designer for Environmental protection

- 1.4.	Licence of Responsable Designer and valid certificates of membership in IKCG

- 1.5.	Statement from the responsible designers that the technical documentation is made in accordance with the regulations

- 1.6.	The insurance policy for the Company responsible for the preparation of technical documentation

- 2.	TEXTUAL DOCUMENTATION

- 2.1	Basic characteristics of the project

- 2.1.1	Current state

- 2.2	Prescribed type and scope of work on the bridge M40

- 2.3	Legal framework

- 2.4	Potential impacts on the environment

- 2.5	Environmental protection measures

- 2.5.1	Safety measures during construction

- 2.5.2	Technical measures in the process of exploitation

- 2.5.3	Measures in the event of an accident

- 2.5.4	Other measures

- 2.5.5	Summary view of expected environmental effects with measures of diminishment

- 2.6	Construction waste management guidance

- 2.6.1	Type, characteristics and origin of waste produced and to be treated or deferred

- 2.6.2	Measures for reducing the amount of waste and diminishing its negative influence

- 2.6.3	Waste management methods

# EXECUTIVE SUMMARY IN ENGLISH ________________________________________________

- CONTENT OF EXECUTIVE SUMMARY IN ENGLISH

EXECUTIVE SUMMARY IN ENGLISH ________________________________________________	3

1.	Legal framework	5

2.	Environmental protection measures	7

2.1.	Safety measures during construction	7

2.1.1.	Noise protection measures	7

2.1.2.	Measures for the protection of soil, underground and surface waters	7

2.1.3.	Air protection measures	9

2.1.4.	Flora and fauna protection measures	9

2.2	Technical measures in the process of exploitation	9

2.2.1.	Air protection measures	9

2.2.2.	Measures for the protection of soil, underground and surface waters	9

2.2.3.	Flora and fauna protection measures	9

2.2.4.	Noise and vibration protection measures	10

2.2.5.	Protection measures against non-ionizing radiation	10

2.3	Measures in the event of an accident	10

2.3.1.	Fire protection measures	11

2.4	Other measures	11

2.4.1.	General environmental protection measures	11

2.4.2.	Administrative environmental protection measures	12

2.5	Summary view of expected environmental effects with measures of diminishment	12

3.	Construction waste management guidance	13

3.1	Materials that will be turned into waste during the reconstruction of the concrete railway bridge	13

3.2	Measures for reducing the amount of waste and diminishing its negative influence	16

3.2.1.	The list of measures taken to reduce waste production	16

3.3	Waste management methods	18

- Legal framework

LAWS

- Environmental law ("Official Gazette of the Republic of Montenegro", 52/16)

- Nature protection law ("Official Gazette of the Republic of Montenegro", No. 54/16)

- Law on strategic environmental impact assessment ("Official Gazette of the Republic of Montenegro", No. 80/05 and "Official Gazette of the Republic of Montenegro “, No. 40/11, 27/11 and 52/16)

- Environmental impact assessment law ("Official Gazette of the Republic of Montenegro “, No. 80/05 and  "Official Gazette of the Republic of Montenegro “, No. 40/10, 40/11, 27/13 and 52/16)

- Law on integrated prevention and control of environmental pollution ("Official Gazette of the Republic of Montenegro “, No. 80/05)

- Law on waste management ("Official Gazette of the Republic of Montenegro”, No. 64/11, 39/16)

- Law on chemicals ("Official Gazette of the Republic of Montenegro”, No. 51/17)

- Law on air protection ("Official Gazette of the Republic of Montenegro”, No. 25/10, 40/11 and 43/15)

- National parks law ("Official Gazette of the Republic of Montenegro”, No. 28/14 and 39/16)

- Law on environmental noise protection ("Official Gazette of the Republic of Montenegro”, No. 28/11, 28/12, 62/13, 1/14, 9/15 and 2/18)

- Law on mountain trails ("Official Gazette of the Republic of Montenegro ", No. 51/08 and 53/11)

- Law on forests ("Official Gazette of the Republic of Montenegro ", No. 74/10, 40/11 and 47/15)

- Law on animal welfare protection ("Official Gazette of the Republic of Montenegro ", No. 14/08, 40/11 and 47/15)

- Water resources law ("Official Gazette of the Republic of Montenegro ", No. 27/2007 and "Official Gazette of the Republic of Montenegro", No. 32/11, 48/15, 52/16, 55/16, 2/17 and 80/17)

- The railway law ("Official Gazette of the Republic of Montenegro", No. 27/13 and 43/13)

DECREES

- Decree on projects for which an environmental impact assessment is carried out ("Official Gazette of the Republic of Montenegro", No. 80/05 and "Official Gazette of the Republic of Montenegro", No. 47/13)

- Decree on classification and categorization of surface water and groundwater ("Official Gazette of the Republic of Montenegro “, No. 2/07)

- Decree on establishing network of measuring points for air quality monitoring ("Official Gazette of the Republic of Montenegro “, No. 44/10 and 13/11)

- Decree on determining the pollutant types, the limit values and other air quality standards ("Official Gazette of the Republic of Montenegro “, No. 25/12)

- Decree on activities which influence or may influence air quality ("Official Gazette of the Republic of Montenegro “ No. 61/12)

- Decree on the list of hazardous substances, allowed quantities and criteria for categorization of hazardous substances ("Official Gazette of the Republic of Montenegro", No. 5/11)

- Decree on substances which damage the ozone layer and alternative substances ("Official Gazette of the Republic of Montenegro",  No. 5/11)

- Decree on the maximum degree of national emissions of certain air pollutants ("Official Gazette of the Republic of Montenegro", No. 3/2012)

- Decree on the method and procedure for establishing the system for taking over, collecting and packaging waste and the operation of the system ("Official Gazette of the Republic of Montenegro", No. 42/12)

- Decree on methods and conditions of waste storage ("Official Gazette of the Republic of Montenegro", No. 33/13)

- Decree on the national list of indicators of environmental protection ("Official Gazette of the Republic of Montenegro", No. 19/13)

REGULATIONS

- Regulation on the content of the environmental impact assessment study ("Official Gazette of the Republic of Montenegro", No. 14/07)

- Regulation on the content  of the documentation submitted with the request for determining the scope and content of the environmental impact assessment study ("Official Gazette of the Republic of Montenegro", No. 14/07)

- Regulation on the content  of the documentation submitted with the request for deciding on the need  for the environmental impact assessment  ("Official Gazette of the Republic of Montenegro", No. 14/07)

- Regulation on detailed requirements which the manager of the protected natural asset must meet ("Official Gazette of the Republic of Montenegro", No. 35/10)

- Regulation on detailed content and the method of keeping and maintaining the cadastre of environmental pollutants ("Official Gazette of the Republic of Montenegro", No. 43/10)

- Regulation on detailed content and the method of keeping and maintaining the register of protected natural assets ("Official Gazette of the Republic of Montenegro", No. 79/09)

- Regulation on allowable quantities of hazardous and harmful substances in the soil and the methods for their testing ("Official Gazette of the Republic of Montenegro", No.18/97)

- Regulation on noise limit values in the environment, the method of determining noise indicators, acoustic zones and the methods for the harmful effect of noise assessment ("Official Gazette of the Republic of Montenegro", No. 60/11)

- Regulation on the methods and conditions for air quality monitoring ("Official Gazette of the Republic of Montenegro", No. 21/11)

- Regulation on the methods and conditions for measuring wastewater quantities which is discharged into the collection systems ("Official Gazette of the Republic of Montenegro", No. 24/10)

- Regulation on the methods and measuring process of water quantities at the water catchment ("Official Gazette of the Republic of Montenegro", No. 24/10)

- Regulation on the methods and conditions for air quality monitoring ("Official Gazette of the Republic of Montenegro", No. 21/11)

- Regulation on the contents and methods of providing annual information about air quality ("Official Gazette of the Republic of Montenegro", No. 27/12)

- Regulation on dealing with waste oils ("Official Gazette of the Republic of Montenegro", No. 48/12)

- Regulation on construction waste management, methods and processing of construction waste, methods and terms of  asbestos cement construction waste disposal ("Official Gazette of the Republic of Montenegro", No. 50/12)

- Regulation on detailed methods and required documentation for issuing permits for allowable emissions of air pollutants ("Official Gazette of the Republic of Montenegro", No. 25/13)

- Regulation on waste incineration and/or co-incineration ("Official Gazette of the Republic of Montenegro", No. 33/13)

- Regulation on the method and process of measuring emission from stationary sources ("Official Gazette of the Republic of Montenegro", No. 39/13)

- Regulation on keeping the register of issued permits for waste processing and/or disposal, and the register of collectors, carriers, traders and waste mediators ("Official Gazette of the Republic of Montenegro", No. 47/13)

- Regulation on waste classification and the waste catalogue ("Official Gazette of the Republic of Montenegro", No. 59/13)

- Environmental protection measures

  - Safety measures during construction

Pursuant to the occupational safety law ("Official Gazette of the Republic of Montenegro, No. 34/14), it is necessary to take measures of occupational safety in order to prevent hazards which might occur during the process of object construction.

An organization which is registered for the appropriate type of activity must be hired for the execution of the construction work. At the construction site there must be a person authorized for work management. That person is required to have passed the professional exam and also has to meet other requirements in accordance with the law on spatial planning and facility construction. Organization, the authorized person and all other individuals involved in construction work, have to comply with the laws, regulations, standards and norms for the type of the activity they are dealing with.

It is the investor's duty to provide the Study of construction site design. The study must be signed by a qualified person who gathered documentation, and certified by the investor's representative or by supervisory services, after which the work may commence. It is the investor's duty to provide expert supervision for the on-site execution.

The environmental action plan will have been prepared within 15 days after the date of commencement of the work.

Prior to the commencement of work the exact position of all installations must be determined, and take the necessary measures to avoid damaging them, and to prevent injuries of workers and other individuals at the construction site.

During the reconstruction of the bridge No. 41 in km 332+539,76 Vrbnica - Bar railway, it is necessary to take a number of measures as to minimize possible influence on the environment.

    - Noise protection measures

Both heavy construction machinery and construction mechanization traffic used for construction work are noise sources.

The contractor is required to use modern equipment with noise suppression systems and to comply with usual working hours during the work day. Maximum accuracy and functionality in order to minimize noise and vibration emissions are ensured through regular (periodical), and extraordinary (if needed) technical inspections of the equipment and vehicles.

    - Measures for the protection of soil, underground and surface waters

Protection measures include all of the procedures necessary to bring quantified negative impacts within the acceptable limits, as well as procedures to minimize impact during the construction phase:

- Strict protection of all parts of the terrain outside of the immediate work area, which means that outside of the railway route existing surfaces cannot be used as permanent or temporary landfill sites for various materials, borrowing sites, parking and machine repairment plateaus;

- For waste waters that will emerge upon sand blasting of the damaged concrete, and for all possible liquid waste which will appear as a consequence of the works directly on the bridge, it is necessary to organize direct collection, not allowing anything to pour out into the road under the bridge, or into the surrounding area. The proposed measure involves direct collection of such waste at the place of its origin into a specially designed receptacle which will be positioned directly under the current working position, and shall be re-positioned as the work progresses.

- Used water from the construction site should be taken in, collected and treated appropriately. Waste waters cannot possibly be discharged without prior treatment;

- Opening access roads to certain parts of the construction site is prohibited.

- The surface layer of fertile soil (humus) in the intake should be dug out and deposited outside of the immediate working area for later use for biological protection and recultivation of construction and other sites. Temporary landfill must not jeopardize original natural soil and cannot be placed where it would be additionally moistened.

- All materials from the excavation, which will not be immediately used for construction, must be stockpiled at pre-arranged locations protected from erosion;

- Bulk material storage must be protected from the wind as to prevent scattering, and sheltered from all kinds of precipitation. Oil and other hazardous materials storage and management is conducted in a controlled manner, including soil and water anti-contamination measures. Drums and tanks with the aforementioned hazardous substances are to be stored under eaves or in indoor premises with ventilation in impermeable bund walls (liquid collectors);

- Strict control of mechanization access to water courses and other surface waters is absolutely necessary;

- Stationing machinery only in pre-arranged locations. Special measures of soil protection from oils and petroleum products must be taken at those parking places. In the event of soil contamination from leaky oils or any other kind, it will be required to remove the layer of soil in question and to transport it to the land fill (pursuant to "Waste catalogue").

- Define proper storage and management of petroleum products and paints. The entire handling of petroleum and its derivatives in the construction process must be conducted in a pre-arranged location and with maximum precautionary measures in order to avoid any spills or soil and water contamination. A bund wall is to be designed under the fuel tank, which solves the problem of penetrating pollutants into the surrounding ground and groundwater. All petroleum and its derivatives packaging and waste materials from machinery maintenance must be collected and stored until being handed over to an authorized operator for further treatment.

- Waste generated during the construction phase should be handled in accordance with the Waste management plan.

- Maintenance, refueling and construction machinery clean up is to be conducted at pre-arranged locations before the commencement of work, farther from water courses and canals which lead the water therein (minimally 50 m). It is necessary to set up a protection that would prevent the spillage of fuel on the surrounding terrain, and it is equally as necessary to foresee the discharge of oily waste waters to the oil and grease separator that will be temporarily placed within the construction site, with deposits that will be collected and stored until being handed over to authorized operators for further treatment;

- Upon completion of work on the surface of construction sites, it is necessary to repair the roads and the degraded temporarily occupied areas, i.e. bring it back to a purposeful state for the use of space;

- Prescribe a ban on washing the machinery and vehicles in the working area, as well as washing concrete mixers and uncontrolled removal of the remaining parts of the concrete mass on any surface in and out of the rail track;

- In the event of concrete surplus at the construction site, the contractor should prepare framework for concrete sleeper - scaffolding base, crane counterweights, building boards etc. If concrete does not get installed for some reason, concrete mixer shall be taken back to the base, where the supplier rinses it with powerful water jets, thus separating an aggregate which can further be used in production, or involved in other procedures. If concrete mixer rinsing cannot be avoided on-site, it is the contractor's duty to prepare a special foil-coated pit, or in some other way protected from soil pollution, where surplus concrete will be poured into;

- Prevent water leakage which is the result of mixer rinsing, as high pH of such water can degrade the environment;

- Iron waste and other metals should be collected and handed over to an appropriate authorized organization for further treatment;

- Spillage of any hazardous substances must be avoided in the immediate vicinity of the water courses. In this respect, the contractor should require to have machinery which uses biodegradable lubricants as well as biodegradable gear oil, in order to minimize pollution during the execution of work;

- Prevent driving in of the machinery into rivers, streams, and on their banks;

    - Air protection measures

During the execution of construction work it is necessary to take a series of measures in order to minimize negative impact on the air quality:

- In order to prevent the uncontrolled dispersion of construction materials by transportation vehicle, it is necessary to clean the vehicle before it enters public areas (if the transport is carried out by local roads), as well as the obligatory covering or moistening of the material being transported in order to avoid its dispersion;

- Regular spraying of the surfaces from which the dust can be blown is to be conducted in dry and windy weather.

- Ensure technical safety of mechanization through regular and extraordinary (if needed) technical inspections of harmful gases emission norms.

    - Flora and fauna protection measures

The construction phase involves measures which must be taken into consideration in order to minimize the impacts on the flora and fauna of the area in question. Construction site is to be organized at the minimal surface required for its functioning, and when choosing a site, make sure that it is not a place with distinctive flora and fauna characteristics, in order to avoid unnecessary loss of biotope.

Do not perform work at the time of bird nesting.

  - Technical measures in the process of exploitation

Starting from the fact that every human activity causes disturbances in the environment, the following measures and procedures are suggested, as to minimize the potential risks:

    - Air protection measures

Exploiting a rail strip that is electrified does not significantly impair air quality in the area in question. Therefore, protection measures are not necessary.

    - Measures for the protection of soil, underground and surface waters

Officially installed containers for municipal, recyclable and hazardous (electronic) waste.

A somewhat more pronounced impact on the quality of soil, underground and surface waters occurs when weeds are treated with herbicides. In order to minimize this impact, measures of protection against the use of herbicides have been defined.

    - Flora and fauna protection measures

In protected areas and their protected zones it is forbidden to:

- form pits and deposit waste materials;

- temporarily or permanently dispose hazardous substances;

- install any temporary facilities or materials for the sake of railway line works;

- park and service mechanization, manipulating with fuels and oil lubricants

    - Noise and vibration protection measures

For the railway superstructure, open tracks and in the stations, use elastic fastening tools, whereas on bridges, overpasses and through urban areas use elastic mattresses under sheeting, which will contribute to possible noise alleviation.

    - Protection measures against non-ionizing radiation

After the construction i.e. installation of an object containing the source of non-ionizing radiation, and prior to the issuance of a license for the commencement of operation or the permit for use, it is necessary to do the test, that is the measurement of the level of the electromagnetic field in the vicinity of the source.

For the purposes of the first test, the user may allow the source of the electromagnetic field to work for a probation period not longer than 30 days, or in case of telecommunication objects, measurement can be conducted in the process of technical examination.

The body in charge of carrying out technical inspection, i.e. of issuing a license for the commencement of operation or use permit for an object containing a source of non-ionizing radiation of special interest, may put this source into operation if it is determined by the measurement that the level of the electromagnetic field does not exceed the prescribed limit values and that the built in (installed) object will not endanger the environment during work.

Electromagnetic field measurements to be conducted every 4 years.

- Contact network

Protection against accidental contact of parts under voltage is achieved by applying the prescribed distance from voltage lines, insulation, protective bulkheads, remarks and labels. Short-circuit protection in the 25 kV network is achieved by distant KM protection and vacuum switches in the EVP extraction fields. The protection against excessively high voltage is achieved by grounding supporting structures of KM and all other metal constructions, 8m from the axis of the track of the earthed rail near the track on the KM return line in accordance with the regulations, as well as the reliable and rapid exclusion of the voltage in KM at if there happens to be a fault. Protection against improper handling is ensured by the organization of the maintenance service of the KM and the application of appropriate instructions, policies and manuals. Fire and explosion hazards are eliminated by using non-flammable and non-combustible standard equipment. The use of electric traction in areas exposed to explosive mixtures is not permitted. Protection against electromagnetic impact on the surrounding pipelines is achieved by the use of SS devices and TK devices and cables in which the design and construction contain appropriate protective measures, cables with a small reduction factor. The magnitude of the electric field and the magnetic induction do not exceed the permissible value even in the most critical points that could be available to personnel, or the passengers, so that their action does not have any adverse effects.

- Signaling and security devices and telecommunication facilities

Cables are designed in such way that their PE external protection is insoluble in water, while mechanical and electrical protection from Al and Fe in case of a direct contact with groundwater does not result with a harmful chemical reaction. Radio devices will be used in accordance with the conditions prescribed by the Agency for Electronic Communications and Postal Services, so as not to disturb other users of radio frequencies. Fire alarms are manufactured, transported and installed in accordance with relevant regulations, so that no harmful radiation can occur. The radiation level is such that it can not harm the environment in the normal mode of exploitation.

Power supply of a TK device from an AC voltage of 3x380 V or 220 V / 50 Hz is carried out with the usual protection measures (grounding, isolation, fuses, etc.). The DC power supply is performed at a voltage of 48 V and less, which is harmless even in case of direct contact. Either way, it is important that measures taken on existing plants continue to be implemented. It should also be noted that the increase in efficiency and functionality of the SS and the TK system facilitates functionality of railway transport, which has direct consequences on reducing the environmental impact.

  - Measures in the event of an accident

Considering the fact that there is a risk for accidents when vehicles which transport hazardous substances are involved, it is a prerequisite to provide special protection measures. A series of measures planned in the context of the general environmental protection have their full meaning and provide significant reliability of the whole system in cases of accidental pollution.

Having in mind the importance of the area in which the bridge is located, it is necessary that the measures for the evacuation and neutralization of toxic substances are foreseen in the planning and design phase of the reconstruction of the object. In case of an accident with dangerous cargo (in powder form, granular or liquid state), traffic must be halted, a specialized service in the nearest place or maintenance base must be contacted, in order to undertake removal operation of the dangerous cargo. In the event of derailing of a vehicle that  carries dangerous cargo subsequent discharge of the cargo, there is a risk of contamination of soil and groundwater.

In order to avoid lasting negative effects on ecological potentials, pollutants must be removed before they come into contact with the body of water. The journey time of the dangerous substance depends on the water-permeability quotient of the soil and groundwater levels.

Remedial action refers to the following activities:

- stop the leakage of a dangerous substance;

- limiting the contaminated zone in order to prevent its further spread;

- gathering pollutants in intervention vessels or tanks;

- setting up temporary dams in waterways which the dangerous substance has reached, or is about to reach;

- preventing leakage of pollutants in the pipeline of the water source and sewer;

- use of various means for decontamination of the terrain and remediation at the leakage of dangerous substances. All materials collected in this way are treated according to special regeneration procedures or are deposited into landfills designed for that purpose. If contamination spreads out of the railway strip regardless of the taken safety measures, one of the methods of remediation (ex situ or in situ) must be conducted, either for soil or groundwater, if contact has been made.

    - Fire protection measures

Accuracy checks of installations must be conducted at least twice a year by an authorized legal entity (by the Ministry), in accordance with the manufacturer's technical regulations and instructions through periodic controls. The record of performed checks shall be kept alongside data about the performed inspection with an issuance of the expert report. Employees who perform these checks are required to have passed the professional exam.

  - Other measures

    - General environmental protection measures

General environmental protection measures include global findings from this domain which are appropriate for the global strategy and local spatial conditions and characteristics of the railway line.

- All activities proclaimed as part of the general development policy at the level of the Republic of Montenegro, which are concretized in paramount planning documents, should be taken into account in terms of rational environmental management for a specific investment project;

- Provide preconditions for constant monitoring of the environmental state on the railway by providing data obtained through measurement;

- Provide preconditions for the continual maintenance of the railway;

    - Administrative environmental protection measures

Administrative environmental protection measures include various activities in order to administratively regulate certain occurrences, which, if not regulated on time, could have negative consequences which can hardly be curbed within acceptable limitations. These measures include the following activities:

- Provide instruments within the framework of the consent issued by the responsible institutions of the Republic (responsible Ministries) to carry out permanent control in the course of the execution of works in terms of possible environmental impacts,

- Provide instruments, within the framework of contractual documentation that the investor has established with his contractors, on the necessity of complying with all of the  prescribed protection measures in the execution of works phase,

- Provide instruments to make sure that in the realization of tasks in the domain of construction and exploitation those entities which have professional staff to fulfill defined tasks in the domain of environmental protection are hired;

In addition to the defined environmental protection measures, it is also necessary to undertake a number of other procedures and actions that are mostly organizational in nature, as well as control of the work of the employed personnel in the field of activities which can influence environmental degradation.

  - Summary view of expected environmental effects with measures of diminishment

Table   Expected environmental effects and diminishment measures

| Expected environmental measures (caused by bridge reconstruction) | Diminishment measures |
| --- | --- |
| 1. Dust and exhaust gases emissions | - Waste material will be disposed of at previously determined locations (by no means on the surrounding roads and/or pavement) and sprayed with water; - construction material and waste will be covered during transport; - Construction machines' "idle" work will be minimized |
| 2. Noise and vibration emission | - Construction site noise will be time-limited, which will minimize the impact to the surrounding area; - Generator engines, compressor engines and similar equipment will be placed as far as possible from the settlement. |
| 3. Underground and surface water pollution | - Measures will be taken to ensure that construction, waste or any other material from the site does not reach the surrounding watercourses or drainage systems; - Construction machines and tools will be washed in places where they will not pollute the surrounding watercourses. |
| 4. Landscape protection | - Cutting of wood, shrubs and other important landscape elements should be minimized as much as it is possible. |
| 5. Waste management, including poisons | - Collected waste from the construction site will be stored at pre-determined locations prior to disposal; - Materials that can be recycled or reused will be separated from the rest of the waste and adequately stored; - Waste containing toxic or potentially hazardous substances will be disposed of in specially marked containers, out of which it is impossible to leak out and contaminate soil and waters. |

- Construction waste management guidance

  - Materials that will be turned into waste during the reconstruction of the concrete railway bridge

Table  types of waste by activity

| Waste type | Activity | Possible problems |
| --- | --- | --- |
| 1. Detritus (concrete blocks, concrete remains and other construction materials) | - Damaged parts of the construction remioval - Loose material removal - Channel covers for installation removal | - Soil pollution, - Temporary disposal - Permanent disposal, - Dust emission during detritus handling |
| 2. Dirty sandy water with anti-corrosive, metal waste residue, liquid mortar, liquid concrete residuals, sprayed concrete, polymer concrete | - Concrete sand blasting - Sprayed concrete repairment - Thin layer of polymere concrete production, - Crack injection - Protective layer based on repair mortar production | - Soil pollution, vegetation under the bridge |
| 3. Bitumen based waste (hydroisolation) | - Replacement of hydroisolation, pavement construction, hiking trails and installation channels | - Temporary disposal - Permanent disposal |
| 4. Pavement material (gravel) | - Pavement material replacement | - Dust emission during the construction - Temporary landfill or containers |
| 5.Timber (rail ties) | - Rail ties removal | - Wood material pollution by means of different agents for weed destruction and/or impregnation |
| 6.Metal waste (rails, vertical outlets, bridge dilatations, dismantled fence residue, sawdust, screws, tiles and shins, pads, anchors, armature...) | - Rail replacement, - Fence replacement - Vertical outlet replacement - Bridge dilatations replacement - Replacement of the connector between the electrification pillar and the console - Removing old pipes from the installation network etc. | - Temporary disposal - Permanent disposal |
| 7. Utilization and spillage of hazardous liquid materials (various types of oil, liquid fuels) | - Accidental fuel spills in the process of vehicle manipulation - Spillage and leakege of oils and fuels | - Emission in the air, - Soil and water pollution, - Possible fires, - Danger of accidents, - Terrain remedial measures at the point of spillage or leakege of dangerous liquids |
| 8. Formwork, lagging | - Concrete work | - Temporary disposal - Permanent disposal, |
| 9. Packaging (plastic, wooden pallets, glass, foil, cardboard, wood chips etc) | - Delivering material and equipment | - Temporary disposal - Permanent disposal, |
| 10. Various types of plastics (waste material from the equipment) | - Fitting in hydroisolation stripes etc. | - Soil contamination, - Temporary disposal - Permanent disposal, |
| 11. Vegetation | - Woodcutting - Cleaning the terrain surface for the preparation of a temporary construction site and daily waste disposal site | - Dust emission, - Fire hazard, - Traffic impediment |

Municipal waste, as well as all other types of waste, should be separately collected and temporarily disposed of at an appropriately marked location.

Balast bed  is made of road metal. Pollution occurs by treating stripes of the track with herbicides, fungicides and pesticides, by releasing faeces on the railway (from the wagons' sanitary knots), by dripping oil, grease and lubricants from trains and wagons, by spilling and scattering of transported cargo. Replaced bulk material will be removed from the construction site and transported to locations which have enough capacity to take it in, without damaging surrounding areas and with the possibility to re-use or transport it. In case of the bridge No. 41 at km. 332+539,76 Vrbnica-Bar railway line, replaced bulk material will be located at Kolašin train station (km 340+600). Transportation of the equipment, machines and materials from Kolašin Station to the bridge will be by the rail vehicles, and transport the people by the road to the bridge.

By the rule book on categories of waste with lists, road metal can be classified as hazardous or non-hazardous waste. In order to be classified according to the waste catalogue, Montenegro Railways have to perform its characterization, which will give out information about the possible content or concentration of substances which make it hazardous. Depending on the level of contamination, it can afterwards be decided upon its disposal or further use.

There are numerous possibilities how to use old road metal. It is most commonly used as replacement material in maintaining low-level railroads. Road metal can also be used for unrelated purposes (such as pit clogging, or at landfills). Finally, one of the contemporary methods is biological remediation of road metal, by enviromentally acceptable chemical substances.

Railroad ties are coated with creosote oils to prevent further degradation, and are periodically treated with pesticides, fungicides, and herbicides. In the process of replacing worn-out railroad ties, in accordance with the Law on Waste Management, or the Rule book on waste categories with lists, such waste must be adequately taken care of. Considering the fact that wood treated with creosote oil permanently retains characteristics of hazardous substances, after use, Montenegro Railways have to classify wooden ties as hazardous waste and temporarily stored on the railway land until the conditions for permanent disposal are created.

Since the majority of the wood protection agents (both organic and inorganic) falls in the hazardous waste category, it is possible that the wood treated with these materials represents hazardous waste. Therefore, sample analysis before the commencement of work is advocated. Until 2005, Montenegro Railways carried out supply of timber materials impregnated with creosote type B oil, and as of 2006, timber with impregnated ecological oils of type C has been supplied, in order to reduce the negative impact on the environment

Re-using rail ties is possible with the provision of certain technical terms and conditions. Usable rail ties can be sold for industrial tracks, while old rail ties can be used to protect the tracks from torrents, mudslides, and to make auxiliary tracks for railway workers, to unload heavy machinery, and so on.

At the location where the old rail ties will be deposited, it is necessary to provide water impermeable conditions because the rail ties have been treated with oil, and the contact with water can cause contamination of the surrounding soil.

Old rail ties will be temporarily deposited in Kolašin and Trebaljevo station, on an adequately secured station plateau, on a strip of land owned by the railway.

Wooden waste will appear on the site during preparatory works, since there is a considerable amount of trees and shrubs that will be removed during terrain shredding. Wooden waste also comprises packaging that will be used during reconstruction (e.g. wooden pallets). Wooden waste can be sorted, cut out and, upon collection from the location, handed over to the local population to be used for heating, provided it has not been oiled or covered with paint.

In addition to the waste that will be generated during the execution of works, and which is specific for particular locations along the railway line, there is also waste which appears in all phases and through all activities, such as: different packaging materials, various types of plastic and glass bottles, cans, barrels and so on. For all these types of waste it is necessary to provide a temporary place for disposal on the construction site, until it is eventually handed over to the company in charge of further treatment. When collecting and disposing of plastic waste (bottles, drums, etc.), special attention is to be paid to the plastic packaging of oils, lubricants, fuel, etc. The aforementioned packaging is not treated as plastic, it is treated as hazardous waste.

Waste oil  originating from mechanization which will be used during the reconstruction of the bridge will also require special treatment. The regulated landfill, in this context, implies water impermeability and inability of water to penetrate the landfill. This can be achieved by laying the foils of appropriate quality on the ground, where the deposit and re-covering at the end of the deposit will be carried out, in order to prevent contact of the deposited waste with rain.

Metal waste  will predominantly consist of superstructural elements (including rails, fasteners, etc.), as well as drainage verticals, bridges dilatations, waste from dismantled fences, shavings, bolts, tiles and ties, anchors, fittings, etc. This waste will be stored in working units of Montenegro Railways, in the facilities already intended for this function.

  - Measures for reducing the amount of waste and diminishing its negative influence

- Containers will be placed at Mojkovac station during the execution of the works for each of the specific types of waste. For wastewaters that will emerge during sandblasting of damaged concrete, as for all possible liquid waste that will appear as a result of works directly on the bridge, it is necessary to organize direct immediate collection, thus preventing any wastewater to end up onto the road, or on the surrounding strip of land. The proposed measure is direct collection of this type of waste at the place of origin in the built-in receptacle that will be placed directly below the current position of the works and move forward as the work progresses.

- The collected waste from the site will be disposed at previously identified locations at the Trebaljevo station before being disposed of to the landfills.

- Recycable materials that can be re-used will be separated from the rest of the waste and adequately stored;

- Waste containing toxic or potentially dangerous substances will be disposed of in specially marked containers within the temporary construction site, to prevent leakage and soil and water contamination;

- Given that different types of waste require different methods of disposal, it is necessary to give instructions on how to dispose of various types of waste. There will be specifically marked containers for metal waste, others for packaging, and so on.

- It is the contractor's duty to make a decision on the appointment of a waste management person during the execution of the works. Those companies the contractor signs a contract with will be responsible for the final treatment and care of the waste.

- Material deposit is not allowed in the river bed and by the waterfront. All materials should be handled in a way that does not disturb natural waterways regime. Measures will be taken to ensure that the construction waste or any other material from the site does not reach the surrounding watercourses or drainage channels;

- Waste material will be sprayed with water to prevent dust emissions;

- Wagons for the transport of building material and waste will be covered for protection;

    - The list of measures taken to reduce waste production

The waste management plan contains measures which refer to reducing the amount of construction waste that is being disposed of within certain time limitations. In order to reduce the amount of waste during works, the following things need to be done:

- Wood waste from preparatory work to be disposed of at a particular location that was agreed upon.

- Separate the surface (humus) layer from the remaining excavated soil (to a depth of about 40 cm).

- Arrange the building site in such way that the accomodation, vehicles and the entire mechanization are all strictly functional for the location itself.

- Soil should be sprayed so as not to raise dust when vehicles are passing by.

- Liquid fuels and other liquid substances (adhesives, solvents, etc.) used for the operation of machines must be stored in closed containers and kept at a safe place, preferably in a covered water-impermeable enclosure.

- Carefully plan quantities of material as precise as possible to prevent the excessive accumulation of materials and spillage.

- In the event of unintentional leakage, contaminated soil should either be cleaned or removed to an appropriate location. This kind of waste is taken over by licensed companies for the disposal of hazardous waste.

- Prohibit spillage of liquid waste on the ground as well as uncontrolled disposal of chemical agents intended for use during the work period to reduce soil damage.

- Construction waste should be collected and deposited in an environmentally acceptable way.

- Classify waste at the site of origin in order to facilitate further treatment of waste or recycling.

- Mark sites for the immediate disposal of certain types of waste (soil, sand, concrete residues, wooden boards, plastic containers, cardboard and paper packaging, etc).

- Make instructions for handling various types of construction waste during the execution of the works.

- Display instructions on waste handling in visible places.

- Determine a particular location for the temporary disposal of hazardous waste (oiled rail ties and oiled road metal etc.).

- All surfaces damaged by construction work after the completion of works should be restored to their original condition or horticulturally arranged.

- Construction waste, such as concrete waste, concrete rail ties, oily rail ties, etc. It is necessary to handle such waste properly, to separate it at the very place of origin, to store and to deposit it further.

- If there are significant changes in the project during the execution of works, the contractor is obliged to update the construction waste management plan. It is important that the responsible person appointed by the contractor immediately fills in the record sheets (on a monthly basis), in form of a table. In that way it will be possible to monitor the type of generated waste, the amount of waste and the contractor, as well as the company that was hired, and we will have clear information which company took over a certain type of waste for better insight in the amount of waste generated in certain stages of reconstruction.

- It is the investor's duty to find a contractor who has a clearly defined and practically validated quality management policy (production, environment, health) and to make the transfer of environmental responsibility and waste management to the contractor through the contract. A responsible person representing the contractor will be appointed to implement the measures from this plan. It is necessary to appoint such person immediately after the selection of the contractor and to notify the controlling body that is responsible for ecological supervision.

  - Waste management methods

Construction work execution technology ensures that all of the waste generated by works on the reconstruction of the railway bridge is transported on a daily basis by rail to permanent landfills in official places. In the event that this is not feasible for any reason, an internal landfill is planned within the enclosure, with a guardhouse, an accommodation facility for the workers and a tool storage area.

Upon completion of the works, the location of the temporary landfill needs to be restored to its original state. All waste will be temporarily disposed of on the land owned by Montenegro Railways.

For temporary disposal of hazardous waste, the contractor shall determine and adjust the space that will be covered and protected from external factors (wind, rain, etc.). Waste that has hazardous waste characteristics should be briefly accomodated and transported further in order to be treated in an environmentally acceptable manner. It is of utmost importance to pay attention to the land on which it will be deposited. For the disposal and further treatment of hazardous waste, it is necessary to sign an agreement with the company which is authorized for taking over such waste.